NOTICE

DISCLAIMER. The information contained in this publication is subject to constant review in the light of changing government requirements and regulations. No subscriber or other reader should act on the basis of any such information without referring to applicable laws and regulations and/or without taking appropriate professional advice. Although every effort has been made to ensure accuracy, the International Air Transport Association shall not be held responsible for any loss or damage caused by errors, omissions, misprints or misinterpretation of the contents hereof. Furthermore, the International Air Transport Association expressly disclaims any and all liability to any person or entity, whether a purchaser of this publication or not, in respect of anything done or omitted, and the consequences of anything done or omitted, by any such person or entity in reliance on the contents of this publication.

© International Air Transport Association. All Rights Reserved. No part of this publication may be reproduced, recast, reformatted or transmitted in any form by any means, electronic or mechanical, including photocopying, recording or any information storage and retrieval system, without the prior written permission from:

Senior Vice President

Operations, Safety and Security

International Air Transport Association

800 Place Victoria

P.O. Box 113

Montreal, Quebec

CANADA H4Z 1M1

IOSA Checklist
ISSM Part I Edition 9

© 2025 International Air Transport Association. All rights reserved.

Montreal—Geneva

# Section 2 —  Flight Operations (FLT)

Applicability Section 2 addresses safety and security requirements for flight operations of eligible operators. To be eligible, an Operator must meet the following criteria: - Commercial and non-commercial passenger and/or cargo operations; - Aircraft with one or more turbine powered and/or multiple reciprocating engines; - One- or two-pilot operations; - IFR and/or VFR operations; - Aircraft below 5,700 Kg (12,566 lb) MTOW. Note: Aircraft above 5,700 Kg (12,566 lb) MTOW will be eligible only for ISSA initial assessment; thereafter, the operator needs to pursue an IOSA registration or aircraft above 5,700 kg MTOW will be assessed as out of scope during next renewal assessment. Aircraft weight category limitations do not apply for non-commercial operations. The standards and recommended practices in Section 2 are applicable only to those aircraft that are of the type authorized in the Air Operator Certificate (AOC) or equivalent document, and are utilized in commercial and non-commercial passenger and/or cargo operations Certain ISARPs are also applicable to ferry flights, test flights and training flights even though they refer to a non-revenue-generating flights, and such application is indicated in a note that is part of the standard or recommended practice. Individual provisions or sub-specifications within a provision that: - Do not begin with a conditional phrase are applicable unless determined otherwise by the Auditor. - Begin with a conditional phrase (“If the Operator...”) are applicable if the operator meets the condition(s) stated in the phrase.

General Guidance The definitions of technical terms used in this ISSM Section 2, as well as the list of abbreviations and acronyms, are found in the IATA Reference Manual for Audit Programs (IRM).

## 1 Management and Control

### 1.3 Accountability, Authorities and Responsibilities

FLT 1.3.6

The Operator shall assign responsibility to the pilot-in-command (PIC) for: - The safety of all crew members, passengers and/or cargo on board the aircraft when the doors are closed; - The operation and safety of the aircraft from the moment the aircraft is ready to move for the purpose of taking off until the moment it finally comes to rest at the end of the flight and the engine(s) are shut down; - Ensuring checklists are complied with. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed documents that assign responsibilities to the PIC. ☐ Interviewed responsible manager(s) in flight operations. ☐ Interviewed flight/cabin crew members. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The intent of this provision is to ensure that the specified responsibilities are assigned to the PIC and such assignment is evident in Operator policies or procedures. Specifications in item i) and ii) may be satisfied by policies documented in, or referenced in, the OM that assign responsibilities to the PIC in a manner consistent with regulations of the State and the intent of the provision. Slight variations in the wording of policies are permissible if the periods of responsibility as specified in each item are addressed by the operator's policies. For example, an operator could assign responsibility to the PIC for the safety of passengers from the time they board the aircraft until they deplane. Such policy would satisfy this provision because it exceeds the period of PIC responsibility as specified in this provision. The specification in item iii) may be satisfied by any policy or combination of policies that assign the responsibility for compliance with standard operating procedures to the PIC.

FLT 1.3.7

The Operator shall ensure, for the duration of each flight, one pilot is designated to act as PIC. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed documents that describe flight crew composition and/or succession of command. ☐ Interviewed responsible manager(s) in flight operations. ☐ Interviewed flight crew members. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The specification of this provision is satisfied if one pilot is designated to act as PIC, regardless of crew configuration or en route crew changes.

### 1.5 Provision of Resources

### 1.6 Documentation System

FLT 1.6.6

The Operator shall ensure documents that comprise the onboard library, as specified in Table 2.1, are carried on board the aircraft for each flight and located in a manner that provides for access by the flight crew.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed the document that describes the onboard library. ☐ Interviewed responsible manager in flight operations. ☐ Interviewed person(s) responsible for flight operations documentation management/control process(es). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Refer to Table 2.1 for specifications related to accessing performance calculations via telecom systems (e.g. ACARS) in lieu of onboard documentation.

### 1.7 Operations Manual

FLT 1.7.1

The Operator shall have an Operations Manual (OM) for the use of personnel in the flight operations organization, which may be issued in separate parts, and which contains or references the policies, procedures, checklists and other guidance or information necessary for compliance with applicable regulations, laws, rules and Operator standards. As a minimum, the OM shall be managed and controlled in accordance with ORG 2.5.1 and be in accordance with specifications contained in Table 2.2. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed operational documents that comprise the OM. ☐ Identified external documents referenced in the OM that contain operational information used by flight crew. ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined selected parts of OM (focus: contents in accordance with Table 2.2). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The intent of this provision is to ensure the flight crew will find all information necessary to perform its functions within the OM, or within another document that is referenced in the OM. The OM is identified as a source of operational information approved or accepted for the purpose by the operator or the State. Guidance and procedures in the OM enable the flight crew to comply with the conditions and limitations specified in the AOC.

FLT 1.7.4

The Operator shall have a process to develop and establish procedures and checklists for use by the flight crew. Such process shall ensure: - Human factors principles are observed in the design of the OM, checklists and associated procedures; - The specific parts of the OM relevant to flight crew are clearly identified and defined; - If applicable, any differences from procedures and checklists provided by the manufacturer(s) are based on operational considerations. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed process used to develop flight crew checklists and procedures. ☐ Identified specific parts of OM relevant to flight crew. ☐ Interviewed responsible manager(s) in flight operations. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Refer to the IRM for the definition of Human Factors Principles. The intent of this provision is to ensure procedures and checklists are developed in a manner that ensures they are useable, identifiable and consistent with manufacturer specifications. Any deviations from manufacturer procedures or checklists are typically based on operational concerns identified by the operator. Human factors principles in document design and checklist usage typically address the following: - Preparation of documentation in a useable format for information presentation, at the appropriate reading level and with the required degree of technical sophistication and clarity. - Improving user performance through the use of effective and consistent labels, symbols, colors, terms, acronyms, abbreviations, formats and data fields. - Ensuring the availability and usability of information to the user for specific tasks, when needed, and in a form that is directly usable. - Designing operational procedures for simplicity, consistency and ease of use. - Enabling operators to perceive and understand elements of the current situation and project them to future operational situations. - Minimizing the need for special or unique operator skills, abilities, tools or characteristics. - Assessing the net demands or impacts upon the physical, cognitive and decision-making resources of the operator, using objective and subjective performance measures. - The specification in item ii) ensures the relevant sections of the OM are clearly identified as the OM can, in some instances, include sections published for flight operations personnel other than flight crew. As such, all OM sections need not be provided to the flight crew (e.g., training syllabi are usually restricted to training/checking personnel).

## 2 Training and Qualification

### 2.1 Training and Evaluation Program

#### General

FLT 2.1.1

The Operator shall have a training and evaluation program, approved or accepted by the Authority, that consists of ground and flight training and, when applicable, evaluations to ensure flight crew members are competent to perform assigned duties. The program shall address traditional and, if applicable, advanced (or alternative) training and qualification, and ensure training and evaluation is conducted for each type of aircraft in the fleet. Such program shall also, as a minimum, address: - Initial qualification; - Continuing qualification; - Re-qualification; - As applicable, aircraft transition or conversion; - Upgrade to PIC; - As applicable, other specialized training requirements, including those associated with operations authorized the AOC. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed flight crew training/qualification program (focus: program includes each type of aircraft in the fleet). ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined training/qualification course curriculum for selected aircraft types (focus: inclusion of applicable training/qualification courses for each aircraft type). ☐ Examined training/qualification records of selected flight crew members (focus: completion of applicable training/qualification courses). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The intent of this provision is to ensure an operator's training program contains the elements necessary to ensure flight crew members are continuously competent to perform assigned duties. The initial qualification process provided to newly hired crew members includes company indoctrination and initial endorsement on company aircraft types. This presupposes that the newly hired crew member already holds a commercial flying license. Initial endorsement training may not be required as part of initial qualification if a newly hired crew member already holds a type endorsement acceptable to both the State and the operator. Company indoctrination training, however, is always considered a part of initial qualification. Continuing qualification includes recurrent or refresher training and also includes any training necessary to meet recency-of-experience requirements. Transition (conversion) training refers to an aircraft type qualification training and evaluation program for each type of aircraft in the fleet and is not required when an operator only utilizes one type of aircraft. Specialized training could include training on a specific type of new equipment (e.g., ACAS) or training for specific operations to meet requirements of the Authority. Training could be outsourced, in which case services typically range from simple dry lease of a training device to delegation of all training to an external organization (e.g., Authorized Flight Training School).

FLT 2.1.4

If the Operator uses distance learning and/or distance evaluation in the flight crew training and qualification program, the Operator shall ensure such training and/or evaluation is monitored in accordance with FLT 2.1.28 and, if required, is approved or accepted by the State. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed regulatory approval, process for monitoring/continual improvement of distance learning in flight crew training/evaluation program. ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined selected distance learning/qualification course development records (focus: monitoring/continual improvement). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Refer to the IRM for the definition of Distance Learning. Distance learning refers to flight crew training or evaluation that is not conducted in a classroom or face-to-face with an instructor or evaluator, but rather is conducted through the use of distributed printed material or electronic media (e.g., Internet, compact disc, etc.).

#### Training Manual

 FLT 2.1.10

The Operator shall have a Training Manual for the use of flight operations personnel, which may be issued in separate parts, that contains the details of all relevant training programs, policies, procedures, requirements and other guidance or information necessary to administer the Operator's Training Program. The Training Manual shall, as a minimum, be managed and controlled as specified in ORG 2.5.1 and be in accordance with specifications contained in Table 2.2. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed flight crew training manual, regulatory approval, content in accordance with Table 2.2. ☐ Interviewed the responsible manager(s) in flight operations. ☐ Examined selected parts of training manual (focus: content includes policies/procedures/requirements, other guidance/information necessary to administer the training/evaluation program). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The training manual typically applies to instructors, evaluators, line–check airmen, flight crew members, training schedulers, simulator operations personnel, administrative support personnel and other applicable flight operations personnel. The training manual may be split among several publications with the relevant parts made easily accessible to the appropriate personnel.

FLT 2.1.28

The Operator shall have processes for ensuring continual improvement of the flight crew training and evaluation program, to include, as a minimum, the monitoring, recording and evaluation of results of successful and unsuccessful flight crew evaluations. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed processes for program monitoring, continual improvement of flight crew training/evaluation program. ☐ Interviewed the responsible manager(s) in flight operations. ☐ Examined selected records of program monitoring (focus: improvements resulting from monitoring). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Flight crew operational non-compliances, training deficiencies and evaluation trends (simulator, aircraft and line operations) are typically used by the training organization for trend analysis and program improvement. Grading scale criteria (e.g. numerical, letter grade) provides a means to accurately identify areas for improvement.

### 2.2 Training Elements

FLT 2.2.7

The Operator shall ensure flight crew members complete Operator familiarization training prior to being assigned to duties in line operations. Such training shall ensure familiarity with: - Duties and responsibilities; - Relevant state regulations; - Authorized operations; - Relevant sections of the OM. (GM) Conformance Applicability. Specific to Aircraft Type; Included in Initial/Transition/ Conversion Training; Included in Recurrent Training/Continuing Qualification; Conformance through AQP/ATQP/EBT. Yes*; Yes; No; No. * This training may be provided as a complete package included in a company indoctrination course or, if applicable, tailored to address requirements that are different from the individual’s previous training..

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed initial training/qualification course curriculum/syllabus (focus: operator familiarization training; definition of subjects addressed). ☐ Examined selected flight crew member training/qualification records (focus: completion of operator familiarization training prior to assignment to line duties). ☐ Interviewed responsible manager(s) in flight operations. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance This provision and many of the ensuing flight crew training provisions contain a Conformance Applicability (CA) Table. Refer to the General Guidance at the beginning of this Subsection 2, Training and Qualification, for a detailed description of the CA Table. Training is applicable to all flight crew members. Many operators refer to this training course as Basic Company Indoctrination.

FLT 2.2.12

If the Operator transports dangerous goods as cargo, the Operator shall ensure flight crew members complete training and an evaluation in dangerous goods during initial ground training and subsequently once during recurrent training within the 24-month period from the previous training in dangerous goods.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed requirement for training/evaluation in dangerous goods in flight crew training/evaluation program. ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined selected initial/recurrent training/qualification course curricula/syllabi (focus: dangerous goods training/evaluation; definition of specific aspects/subjects addressed). ☐ Examined selected flight crew member training/qualification records (focus: completion of training/evaluation in dangerous goods in initial/recurrent training). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Training and evaluation is applicable to all flight crew members. Recurrent training in dangerous goods is typically completed within a validity period that expires 24 months from the previous training to ensure knowledge is current, unless a shorter period is defined by a competent authority. However, when such recurrent training is completed within the final 3 months of the 24-month validity period, the new validity period may extend from the month on which the recurrent training was completed until 24 months from the expiry month of the current validity period. If such recurrent training is completed prior to the final three months (or 90 days) of the validity period, the new validity period would extend 24 months from the month the recurrent training was completed. The curriculum for dangerous goods training for flight crew members will typically address the following subject areas: - General philosophy; - Limitations; - List of dangerous goods; - Labeling and marking; - Recognition of undeclared dangerous goods; - Storage and loading procedures; - Pilot's notification; - Provisions for passengers and crew; - Emergency procedures. Guidance may be found in the IATA Dangerous Goods Regulations (DGR) 1.5, Table 1.5.A.

FLT 2.2.13

If the Operator does not transport dangerous goods as cargo, the Operator shall ensure flight crew members complete training and an evaluation in dangerous goods during initial ground training and subsequently once during recurrent training within the 24-month period from the previous training in dangerous goods.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed requirement for training/evaluation in dangerous goods in flight crew training/evaluation program. ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined selected initial/recurrent training/qualification course curricula/syllabi (focus: dangerous goods training/evaluation; definition of aspects/subjects addressed). ☐ Examined selected flight crew member training/qualification records (focus: completion of training/evaluation in dangerous goods in initial/recurrent training). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Training and evaluation is applicable to all flight crew members. Recurrent training in dangerous goods is typically completed within a validity period that expires 24 months from the previous training to ensure knowledge is current, unless a shorter period is defined by a competent authority. However, when such recurrent training is completed within the final 3 months of the 24-month validity period, the new validity period may extend from the month on which the recurrent training was completed until 24 months from the expiry month of the current validity period. If such recurrent training is completed prior to the final three months (or 90 days) of the validity period, the new validity period would extend 24 months from the month the recurrent training was completed. The curriculum for dangerous goods training for flight crew members is commensurate with responsibilities and will typically address: - General philosophy; - Limitations; - Labeling and marking; - Recognition of undeclared dangerous goods; - Provisions for passengers and crew; - Emergency procedures. Guidance may be found in DGR 1.5, Table 1.5.B.

FLT 2.2.16

The Operator shall ensure flight crew members complete training and an evaluation in subjects associated with adverse weather and/or environmental conditions during initial ground training and subsequently during recurrent training once every three (3) calendar years. - Cold weather operations; - De-/anti-icing policies and procedures as specified in FLT 3.9.6; - Contaminated runway Operations; - Thunderstorm avoidance. (GM) Note: Item ii) is applicable if the Operator conducts flights from any airport when conditions are conducive to ground aircraft icing.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed requirement for training/evaluation in adverse weather/environmental conditions in flight crew training/evaluation program. ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined selected initial/recurrent training/qualification course curricula/syllabi (focus: training/evaluation in adverse weather/environmental conditions; definition of aspects/subjects addressed). ☐ Examined selected flight crew member training/qualification records (focus: completion of training/evaluation in adverse weather/environmental conditions in initial/recurrent training). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Training and evaluation is applicable to all flight crew members. The specifications in this provision are related to the prevention of runway excursions and in-flight loss of control. The intent of this provision is to ensure flight crew members receive recurrent training and an evaluation in the subjects associated with the adverse weather or environmental conditions they may encounter in operations.

FLT 2.2.20

If the Operator conducts flights into areas where English is the primary language of Air Traffic Control (ATC) and whose duties include communication with ATC, the Operator shall require flight crew members to complete an evaluation during initial ground training to demonstrate a sufficient level of English language proficiency that will ensure effective communication during the performance of such duties. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed requirement for English language evaluation for flight crew members that will operate flights/communicate with ATC in areas where the primary language of ATC is English. ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined selected flight crew member training/qualification records (focus: completion of demonstration of English language proficiency). ☐ Examined initial training/qualification course curriculum/syllabus (focus: demonstration of English language proficiency level necessary for effective ATC communications). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The intent of this provision is to ensure a pilot who is required to communicate with air traffic control in English demonstrates a sufficient level of English language proficiency to ensure effective communication during the performance of duties. Such evaluation applies to each operating member of the flight crew, as required by the AFM, whose duties require communication in English with ATC. English proficiency requirements do not apply to flight engineers or flight navigators unless their duties include air/ground communication in English. A State requirement, as part of flight crew licensing, for an individual to demonstrate expert English language proficiency may be used to satisfy the specifications of this provision.

FLT 2.2.26

The Operator shall ensure flight crew members complete training in normal and non-normal procedures and maneuvers during initial training and subsequently during recurrent training once every 12 months. - Pilot Monitoring (PM), Pilot Flying (PF) and other flight crew division of duties (task sharing), if applicable; - If applicable, positive transfer of aircraft control; - Consistent checklist philosophy; - Emphasis on a prioritization of tasks (e.g. “aviate, navigate, communicate”); - Proper use of all levels of flight automation. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed requirement for training in normal/non-normal procedures/maneuvers in flight crew training/evaluation program. ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined selected initial/recurrent training/qualification course curricula/syllabi (focus: training in normal/non-normal procedures/maneuvers; definition of specific elements/subjects addressed). ☐ Examined selected flight crew member training/qualification records (focus: completion of initial/recurrent training in the specified normal/non-normal procedures/maneuvers). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Refer to the IRM for the definitions of Pilot Flying (PF) and Pilot Monitoring (PM). Training is applicable to all flight crew members. The intent of this provision is to set a training interval for normal and non-normal procedures, and additionally to ensure the training manual, curricula, lesson plans, or other guidance associated with such training addresses the specifications in items (i) through (v). Division of flight crew duties, transfer of aircraft control, checklist use and prioritization of tasks are in accordance with the operator's policy for task sharing and as specified in FLT 3.11.18. Elements of training may be accomplished as part of ground, simulator, aircraft or line training. The term Pilot Monitoring (PM) has the same meaning as the term Pilot Not Flying (PNF) for the purpose of applying the specifications of this provision to two-pilot operations. The specification in item iv) refers to the following prioritization of tasks during any normal or abnormal situation or maneuver: - Aviate: fly the aircraft in accordance with restrictions and limitations set forth in the OM; - Navigate: guide the aircraft along the intended or appropriate route; - Communicate: verbalize intentions to other crew members and ATC, as applicable. The term “abnormal” is used to describe a condition or situation (e.g. abnormal airframe vibration, abnormal landing configuration). The terms “normal” and “non-normal/emergency” typically refer to AOM checklists, procedures and/or maneuvers. The term “non-normal” includes AOM emergency checklists and/or procedures (i.e. an emergency procedure is a subset of non-normal). The terms can also be used to describe an event, situation or operation that would be addressed by normal or non-normal/emergency procedures or checklists. When used in this manner, the terms may be separated by forward slash marks (e.g. normal/non-normal/emergency). The term “emergency” used alone refers to declarations and non-AOM procedures.

FLT 2.2.27

The Operator shall ensure flight crew members complete training and, when applicable, an evaluation, that includes a demonstration of competence in normal and non-normal procedures and maneuvers, to include, as a minimum, rejected takeoff, emergency evacuation, engine failure. Such training and, when applicable, evaluation shall be accomplished during initial training and subsequently during recurrent training once every 12 months. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed requirement for training/evaluation including a demonstration of competence in normal/non-normal procedures/maneuvers in flight crew training/evaluation program. ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined selected initial/recurrent training/qualification course curricula/syllabi (focus: training/evaluation in specified normal/non-normal procedures/maneuvers). ☐ Examined selected flight crew member training/qualification records (focus: completion of initial/recurrent training/evaluation in the specified normal/non-normal procedures/maneuvers). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The intent of this provision is to define the basic initial and subsequent recurrent training and evaluation cycles that ensure flight crew members are competent to perform normal and non-normal procedures and maneuvers. It is understood that competence in all potential normal and non-normal procedures may not be demonstrated annually but in accordance with a schedule that is acceptable to the Authority. Training and, when applicable, a demonstration of competence in specified normal and non-normal procedures and maneuvers is applicable to all pilot crew members. Training and, when applicable, evaluation is to be accomplished as part of ground, simulator/aircraft and line training. Line training is in normal procedures/maneuvers only. Such evaluation of competence in the normal and non-normal procedures and maneuvers specified is applicable when such procedures and/or maneuvers are stipulated by the operator and/or State in conjunction with State-approved or State-accepted training courses that require a method of evaluation. Such courses typically include: - Type qualification; - Transition (conversion); - Upgrade to PIC; - Re-qualification; - Recurrent training. Operators that conduct training flights and cannot safely train/evaluate a non-normal procedure or maneuver in an aircraft or in a representative flight training device may demonstrate an alternative means of conformance.

FLT 2.2.28

The Operator shall ensure flight crew members complete practical training exercises: - In the use of emergency and safety equipment required to be on board the aircraft; - That address emergency evacuation and coordination among flight crew members and, as applicable, cabin crew members and/or supernumeraries required for the safety of operations. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed philosophy/requirements for preparing flight crew members for an evaluation in flight crew training/evaluation program. ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined guidance for instructors/evaluators (focus: methodology for providing information to flight crew members in preparation for an evaluation). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The specification of this provision is not intended to preclude flight crews from knowing the city pairs to be flown or the general maneuver requirements prior to the evaluation; however, flight crews would typically not be provided with the exact evaluation scenario. Operators that conduct training flights in an aircraft may divulge as much information about the intended training/evaluation as is necessary to ensure the safety of the planned operation.

FLT 2.2.30

The Operator shall ensure flight crew members complete training in CRM skills, which may be accomplished as part of simulator, aircraft and/or line training, as applicable. Such training shall be completed during initial training and subsequently during recurrent training once every 12 months. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed requirements for training in CRM skills in flight crew training/evaluation program. ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined selected initial/recurrent training/qualification course curricula/syllabi (focus: inclusion of CRM training in simulator/aircraft or during line flight training). ☐ Examined selected flight crew member training/qualification records (focus: completion of initial/recurrent CRM training). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Training is applicable to all flight crew members. This specification is intended to ensure CRM skills are emphasized during and integrated into simulator or aircraft training, as applicable, and line training.

FLT 2.2.32

The Operator shall ensure flight crew members complete training and, when applicable, an evaluation that includes a demonstration of competence, in wind shear avoidance and recovery from predictive and actual wind shear. Such training shall be completed during initial ground and simulator training, and subsequently during recurrent simulator training once every 36 months. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed requirement for training/evaluation/demonstration of competence in wind shear avoidance/recovery from predictive/actual wind shear in flight crew training/evaluation program. ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined selected initial/recurrent training/qualification course curricula/syllabi (focus: wind shear training/evaluation/demonstration of competence). ☐ Examined selected flight crew member training/qualification records (focus: completion of initial/recurrent wind shear training/evaluation. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Refer to the IRM for the definition of Wind Shear. The intent of this provision is to ensure training and evaluation occurs, as applicable, in the maneuvers specified within the intervals specified. Such training and evaluation can occur in conjunction with any State-approved or State-accepted training course. Training and, when applicable, an evaluation in the specified normal and non-normal procedures and maneuvers is applicable to all pilot crew members. Training is accomplished in a representative flight simulator approved for the purpose by the State. Such evaluation of competence in the normal and non-normal procedures and maneuvers specified is applicable when such procedures and/or maneuvers are stipulated by the operator and/or State in conjunction with State-approved or State-accepted training courses that require a method of evaluation. Such courses typically include: - Type qualification; - Transition (conversion); - Upgrade to PIC; - Re-qualification; - Recurrent training. Operators that cannot conform to the specifications of this provision due to the non-existence of a representative flight training device may demonstrate an alternative means of conforming to these specifications. The additional ground and line training and evaluation used to satisfy the specifications of this provision typically include a review of: - Conditions conducive to wind shear; - Effects on aircraft performance; - Indications of wind shear presence; - Avoidance and recovery techniques; - Wind shear case studies or scenarios.

FLT 2.2.33

If the Operator operates the aircraft which is GPWS installed on board, the Operator shall ensure flight crew members complete training and an evaluation, which includes a demonstration of competence in terrain awareness procedures and maneuvers. Such training shall be completed during initial ground and simulator training and subsequently during recurrent simulator training once every 36 months. Such training and evaluation shall include: - Knowledge and conduct of associated procedures; - Response to GPWS alerts and warnings; - The avoidance of Controlled Flight Into Terrain (CFIT). (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed requirement for training/evaluation/demonstration of competence in terrain awareness procedures/maneuvers in flight crew training/evaluation program. ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined selected initial/recurrent training/qualification course curricula/syllabi (focus: training/evaluation in terrain awareness procedures/maneuvers; definition of subjects addressed). ☐ Examined selected flight crew member training/qualification records (focus: completion of initial/recurrent training/evaluation in terrain awareness procedures/maneuvers. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The intent of this provision is to ensure training and evaluation occurs, as applicable, in the maneuvers specified within the intervals specified. Such training and evaluation can occur in conjunction with any State-approved or State-accepted training course. Training and evaluation in the specified normal and non-normal procedures and maneuvers in a representative flight simulator approved for the purpose by the State is applicable to pilot crew members. Training and evaluation of the non-normal procedures and maneuvers specified in this provision cannot be safely accomplished in an aircraft on a training flight. Operators that cannot conform to the specifications of this provision due to the non-existence of a representative flight simulator may demonstrate an alternative means of conforming to these specifications. The additional ground and line training and evaluation used to satisfy the specifications of this provision typically includes a review of: - CFIT avoidance techniques; - CFIT recovery techniques and maximizing aircraft performance; - GPWS alerts and warnings; - CFIT case studies or scenarios.

FLT 2.2.34

If the Operator conducts low visibility operations (LVO), the Operator shall ensure flight crew members complete training and an evaluation that includes a demonstration of competence in such operations, as well as operations with inoperative ground based and/or aircraft equipment. Such training shall be completed during initial ground and simulator training and subsequently during recurrent simulator training once every 12 months. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed requirement for training/evaluation/demonstration of competence in LVO and/or operations with inoperative ground based/aircraft equipment in flight crew training/evaluation program. ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined selected initial/recurrent training/qualification course curricula/syllabi (focus: training/evaluation in LVO and/or operations with inoperative ground based/aircraft equipment). ☐ Examined selected flight crew member training/qualification records (focus: completion of initial/recurrent training/evaluation in LVO and/or operations with inoperative ground based/aircraft equipment. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Refer to the IRM for the definitions of Low Visibility Operations (LVO). Training and evaluation in low visibility operations is applicable to all pilot crew members. For the purposes of this provision, low visibility operations are considered in effect when the Runway Visual Range (RVR) is below 400 m for takeoff and/or below Category I limits for landing. Operators that conduct training flights and cannot safely train/evaluate the specified procedures in an aircraft or in a representative flight training device may demonstrate an alternative means of conformance.

FLT 2.2.42

If the Operator transports passengers or supernumeraries, the Operator shall ensure flight crew members complete security training as approved or accepted by the State, and in accordance with the Operator's security training program as specified in SEC 2.1.1 Flight crew security training should address the following subject areas: - Determination of the seriousness of the occurrence; - Crew communication and coordination; - Policy and procedures associated with flight deck access; - Appropriate self-defense responses; - Use of non-lethal protective devices assigned to crew members for use as authorized by the State; - Understanding the behavior of terrorists so as to facilitate the ability to cope with hijacker behavior and passenger responses; - Situational training exercises regarding various threat conditions; - Flight deck procedures to protect the aircraft; - Aircraft search procedures; - As practicable, guidance on least-risk bomb locations. (GM) Conformance Applicability. Specific to Aircraft Type; Included in Initial/Transition/ Conversion Training; Included in Recurrent Training/Continuing Qualification; Conformance through AQP/ATQP/EBT. Yes*; Yes; Yes (every 36 months); Yes. * This training may be provided as a complete package or, if applicable, tailored to address aircraft type or crew position requirements that are different from the individual's previous flight crew security training..

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed flight crew security training program (focus: approval/acceptance by the State; meets applicable requirements of other states). ☐ Examined flight crew AQP/ATQP/EBT (if applicable): (focus continuing qualification recurrent schedule for flight crew security training). ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined selected initial/recurrent training/qualification course curricula/syllabi (focus: security training is included; required subjects are addressed). ☐ Examined selected flight crew member training/qualification records (focus: completion of security training). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Refer to the IRM for the definitions of Air Operator Security Program (AOSP) and Non-Lethal Protective Device. Flight crew members are directly involved in the implementation of security measures and thereby require an awareness of obligations to the Security Program of the operator. Crew security training would normally be in accordance with applicable regulations and/or the civil aviation security program of the State, and where no regulatory guidance exists, in accordance with the policy of the operator. Security training for flight crew members typically focuses on the need for the flight crew to maintain control of the flight deck. Specific subject areas included in recurrent security training are typically identified and derived from an analysis of actual or likely situations or trends experienced during line operations. Fight deck access as specified in item (iii) would typically include persons authorized for flight deck access as well as procedures for flight deck entry/exit. Flight crew training in self-defense responses as specified in item (iv) typically focuses on ensuring the security of the flight deck and takes into consideration relevant operational factors (e.g. type of operation, phase of flight, aircraft type/configuration, responses by cabin crew members or, if applicable, supernumeraries). Training as specified in item (vi) typically addresses topics or tactics as appropriate for the operator that might be associated with or could be used to facilitate crew-passenger reaction to or interaction with hijackers (e.g. conflict management, use of passive or non-passive cooperation, understanding Stockholm Syndrome, identification of and response to hijacker types/motives). Training exercises as specified in item (vii) are typically interactive in nature, and scenarios or situations (e.g. bomb threat, hijacking, unruly passenger) may be presented using various accepted training methods (e.g. live role playing, table top, computer-based training). Training as specified in item (x) is applicable to aircraft types that have designated least-risk bomb locations. Least-risk bomb locations are typically not identified on all-cargo aircraft. Refer to General Guidance at the beginning of this Subsection 2, Training and Qualification, for explanatory information regarding traditional training program requirements.

FLT 2.2.43

If the Operator conducts passenger flights without cabin crew, the Operator shall ensure flight crew members, complete training and demonstrate competence in the performance of any assigned duties and functions related to passenger cabin safety. (GM) Conformance Applicability. Specific to Aircraft Type; Included in Initial/Transition/ Conversion Training; Included in Recurrent Training/Continuing Qualification; Conformance through AQP/ATQP/EBT. Yes*; Yes; Yes (every 24 months); Yes. * This training may be provided as a complete package or, if applicable, tailored to address aircraft type or crew position requirements that are different from the individual's previous training in duties and functions related to passenger cabin safety..

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed requirement for flight crew training in the performance of assigned duties/functions related to passenger cabin safety. ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined selected initial/recurrent training/qualification course curricula/syllabi (focus: training in performance of assigned duties/functions related to passenger cabin safety). ☐ Examined selected flight crew member training/qualification records (focus: completion of training in performance of assigned duties/functions related to passenger cabin safety). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The training specified in the provision is to be accomplished as part of initial ground, simulator/aircraft or line training. Cabin safety training would typically address: - Aircraft systems and emergency equipment including: Aircraft interior, passenger seats and restraints; Aircraft-specific cabin duties and responsibilities; Emergency exit locations and operation; Emergency equipment locations and operation; Slides, rafts, slide/rafts, ramp slide/rafts, life vests and other flotation devices as applicable. - Cabin safety duties and responsibilities including: Mandatory passenger briefings; Passenger acceptance and handling; The stowage of carry-on baggage; The use of personal electronic devices; Fueling with passengers on board; Cabin safety checks. - Emergency procedures including: Cabin duties assumed in the event of an emergency; Cabin smoke, fumes and fires; Emergency landing (land and water); Planned and unplanned cabin emergency evacuations (land and water); Oxygen administration Medical emergencies and first aid. Cabin safety training elements incorporated into other curricula of the flight crew member training program may satisfy the specifications of this provision. Refer to General Guidance at the beginning of this Subsection 2, Training and Qualification, for explanatory information regarding traditional training program requirements.

### 2.3 Line Qualification

 FLT 2.3.1

The Operator shall have a line qualification program consisting of line training and, where applicable, evaluations, approved or accepted by the State, which ensures flight crew members are qualified to operate in areas, on routes or route segments and into the airports to be used in operations for the Operator. Such program shall: - Be published in the Training Manual or equivalent documents; - Ensure each pilot flight crew member has adequate knowledge of the elements specified in Table 2.5, as applicable to the areas, routes and route segments of intended operation; - Specify qualification requirements for operations in all areas, on all routes or route segments, and into all airports of intended use; - Ensure each pilot flight crew member, prior to entering the line qualification program, has satisfied the applicable recency-of-experience requirements specified in FLT 3.3.7 (i) under the supervision of an instructor or evaluator authorized for the purpose by the Operator and/or State; - Ensure line training and evaluation for each pilot crew member is completed during initial qualification. - Ensure line training and evaluation is completed prior to a pilot crew member being used as a PIC in operations. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed flight crew line qualification training/evaluation program, approved/accepted by the State, specifies qualification requirements for operations associated with areas/routes/route segments/airports used in operations. ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined flight crew line qualification initial/recurrent curricula/syllabi (focus: line training/evaluation in areas/airports of operations; program elements consistent with specifications in Table 2.5). ☐ Examined selected flight crew member training/qualification records (focus: completion of initial/recurrent line qualification training/evaluation). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The intent of this provision is to ensure flight crew members are qualified to conduct routine operations within each theater of operation as defined by the operator. It does not address the additional and specialized knowledge. The specification in item (iv) ensures that a qualifying pilot has the necessary experience to operate as a required crew member in the line training qualification program. Recency-of-experience would typically be established during simulator training in a ZFTT training program or during base training associated with initial aircraft qualification and/or type rating. If there are delays between simulator and/or type qualification training and the commencement of line qualification, recency-of-experience may be lost and would need to be re-established by the operator. The specification in item (iv) may be satisfied by a process integral to the line qualification program or be in accordance with the process defined by FLT 3.3.7. This specification in item (v) applies to all candidates for the position of PIC, to include SIC upgrade candidates and pilots hired directly into PIC positions in operations for the operator. The training and evaluation specified in this provision is accomplished by pilot flight crew members as part of; ground training, simulator/aircraft training or line training.

FLT 2.3.5

If the Operator conducts single-pilot operations, the Operator shall ensure all pilots-in-command conducting single-pilot operations meet the following requirments: - for operations under IFR or at night, have accumulated at least 50 hours of flight time on the class of aeroplane, of which at least 10 hours shall be as pilot-in-command; - for operations under IFR, have accumulated at least 25 hours of flight time under IFR on the class of aeroplane; - for operations at night, have accumulated at least 15 hours of flight time at night, which may form part of the 50 hours of flight time in sub-item i); - for operations under IFR, have acquired recent experience as a pilot engaged in single-pilot operations under IFR, which includes: - at least five IFR flights, including three instrument approaches carried out during the preceding 90 days on the class of aeroplane in the single-pilot role; or - an IFR instrument approach check carried out on such an aeroplane during the preceding 90 days; - for operations at night, have completed at least three take-offs and landings at night on the class of aeroplane in the single-pilot role in the preceding 90 days; and - have successfully completed training programmes that include, passenger briefing with respect to emergency evacuation, autopilot management, and the use of simplified in-flight documentation. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed requirement for PIC. ☐ Examined records of flight operation. ☐ Interviewed responsible manager(s) in flight operations. ☐ Other Actions (Specify) Click or tap here to enter text.

### 2.4 Special Qualification

 FLT 2.4.1

If the Operator conducts flights into special airports and/or areas as designated by the State or by the Operator, the Operator shall ensure each PIC completes training and, if required, an evaluation in the special skills and/or knowledge required to qualify or requalify for such operations. The content of training shall ensure the PIC has adequate knowledge of the elements specified in Table 2.5 as applicable to the areas, routes, route segments and special airports of intended operation. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed requirement for training to qualify/requalify a PIC in special skills/knowledge needed for operations associated with specific areas/routes/route segments/difficult terrain/airports as designated by State or operator. ☐ Examined training curriculum/syllabus used to qualify/requalify PIC to operate over/into special routes/areas/airports (focus: training in special skills/knowledge required for certain operations; program elements consistent with specifications in Table 2.5). ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined selected PIC training/qualification records (focus: completion of training for operations associated with designated special areas/routes/route segments/terrain/airports). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance This provision applies to candidates for the position of PIC, to include SIC upgrade candidates and pilots hired directly into PIC positions in operations for the operator. Training as specified in this provision may include aircraft type-specific elements as applicable to areas of operations, routes, airports, and equipment operated. The specifications of this provision address the training required to operate over difficult terrain and/or into special airports based on a determination, by the operator and/or State, that pilots require special skills or knowledge for such operations. Such training typically addresses routes and/or airports that are over or in areas: - With mountainous terrain, including high terrain, rapidly rising terrain or terrain with steep gradients; - With terrain that contributes to the existence of mountain waves, turbulence, high surface winds, sudden wind changes and/or other atmospheric phenomena that could affect the performance of the aircraft; - Containing topographical variations such as ridgelines, valleys, ravines, fjords or other areas where downdrafts on the leeward or downwind side can make traversing the area or accomplishing a crosswind landing hazardous; - Where the airport, runway and/or approach environment is difficult to identify at night due to surrounding lights; - Where featureless or expansive terrain could contribute to optical illusions during the day or at night; - That are devoid of lighting where airport, runway and/or approach area identification is difficult at night due to lack of visible landmarks; - That are devoid of lighting and sole reference to external or visual cues is insufficient for the maintenance of proper aircraft attitude control; - That require the application of any other specific skills or knowledge, as determined by the operator and/or State.  The specified training may be included as part of initial or continuing qualification under FLT 3.3.10 or conducted independently.

FLT 2.4.3

If the Operator uses flight crew members to concurrently operate aircraft of different types, or operate variants within one type, the Operator shall have qualification processes that are approved or accepted by the State and ensure such flight crew members complete training and an evaluation that emphasizes the differences between aircraft types and variants. (GM) Conformance Applicability. Specific to Aircraft Type; Included in Initial/Transition/ Conversion Training; Included in Recurrent Training/Continuing Qualification; Conformance through AQP/ATQP/EBT. Yes*; Yes; Yes (every 12 months); Yes. * This training may be provided as a complete package or, if applicable, tailored to address aircraft type or crew position requirements that are different from the individual's previous training in duties and functions related to passenger cabin safety..

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed requirement for flight crew training/evaluation in differences between aircraft types/variants (as applicable). ☐ Identified/Assessed flight crew AQP/ATQP/EBT (if applicable): (focus continuing qualification recurrent schedule for training/evaluation in differences between aircraft types/variants). ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined flight crew line qualification initial/recurrent curricula/syllabi (focus: training/evaluation in differences between relevant aircraft types/variants). ☐ Examined selected flight crew training/qualification records (focus: completion of training/evaluation in differences between aircraft types/variants). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Refer to the IRM for the definitions of Aircraft Type and Aircraft Variant (within Type). The intent of this specification is to ensure flight crew members are familiarized with the significant differences in equipment and/or procedures between concurrently operated types or variants. The determination of variant within type is within the domain of the State as part of flight crew licensing. Qualification processes are applicable to all flight crew members used in such operations and as defined in the IRM. Refer to General Guidance at the beginning of this Subsection 2, Training and Qualification, for explanatory information regarding traditional training program requirements.

## 3 Line Operations

### 3.1 Common Language

FLT 3.1.1

The Operator shall ensure the designation of a common language(s) for use by all flight crew members for communication: - On the flight deck during line operations; - If the Operator conducts passenger flights with cabin crew, between the flight crew and cabin crew during line operations; - During flight crew training and evaluation activities. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed requirement for use of common language(s) by flight/cabin crew members. ☐ Interviewed responsible manager(s) in flight operations. ☐ Interviewed selected flight/cabin crew members (focus: awareness/use of designated common language in operations). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance More than one common reference language might be designated. Communication in the designated common language is applicable to all flight crew members, including foreign nationals and expatriates used as flight crew members, instructors or evaluators by the operator. The operator is expected to be in compliance with the common language requirements of the State (e.g. mandatory for operations, a condition for employment or a condition for airman certification), if such requirements exist. If no State requirements exist, the operator is expected to designate an appropriate common operational language for use by flight crew members, as specified in this provision. The existence (and application) of a State common language requirement that satisfies the specifications of this provision relieves the operator of such a designation in operational documentation.

### 3.3 Flight Crew Qualifications

FLT 3.3.1

The Operator shall specify the composition and required number of flight crew members taking into account the type of aircraft, flight crew qualification requirements and flight/duty time limitations.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed requirement/methodology for determining flight crew composition/number of crew members based on aircraft type/crew qualification/flight-duty time limitations. ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined selected flight crew pairing records (focus: crew composition/number consistent with aircraft type/qualifications/limitations). ☐ Observed flight crew scheduling operations (focus: scheduling complies with defined flight crew composition/number of flight crew members based on mission factors). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The intent of this provision is to ensure flight crews are composed of the flight crew members appropriate for the aircraft type and planned operation. As applicable to an operator, crew composition requirements would typically also address the use of relief pilots and/or augmented crews.

FLT 3.3.4

The Operator shall ensure flight crew members will not operate an aircraft unless issued a medical assessment in accordance with requirements of the State; such assessment shall not be valid for a period greater than 12 calendar months. (GM) Note: If authorized by the State, it is permissible to extend the validity beyond 12 months (to preserve the original expiry date) when the medical assessment is renewed up to 45 days prior to its expiry date.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed requirement for flight crew members to have valid medical assessment in accordance with requirements of the State, maximum 12 months validity. ☐ Identified/Assessed tracking/scheduling processes that prevent flight crew members from assignment to flight duty without valid medical assessment. ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined selected flight crew training/qualification records (focus: existence of valid medical assessment). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Requirements of the State and/or an applicable authority that are associated with medical classifications, aircraft types, flight crew positions and/or licensing could require a more restrictive assessment interval than specified in this provision. An applicable authority is one that has jurisdiction over international operations conducted by an operator over the high seas or the territory of a state that is other than the State of the Operator.

 FLT 3.3.5

If the Operator conducts international flight operations, the Operator shall ensure either of the following apply to flight crew members that operate such flights: - The Operator has a method to prevent such crew members from acting as a pilot after having attained their 65th birthday, or - Where laws or regulations of the State do not permit maximum age limits or prescribe a maximum age limit other than that specified in (i), the Operator has a method, which is acceptable to the State and other applicable states, for making a determination that pilot flight crew members are no longer permitted to exercise the privileges of their pilot license in international operations for the operator. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed requirements/restrictions applicable to use of pilot flight crew members that have attained 65 years of age, as applicable. ☐ Identified/Assessed requirements/restrictions applicable to use of pilot flight crew members that have attained the State prescribed maximum age limit. ☐ Identified/Assessed tracking/scheduling processes that ensure assignments of PIC and crew pairing for international flights are in accordance with age limitations/restrictions. ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined selected flight crew training/qualification records (focus: retention of age imitations/restrictions). ☐ Observed flight crew scheduling operations (focus: scheduling accounts for/complies with age/other restrictions that prohibit assignment of flight crew member as pilot in international flight operations). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The intent of this provision is to address pilot flight crew member duty assignments for international operations when the flight crew includes at least one pilot that has attained 65 years of age, or the maximum State prescribed age limit. The specifications of this provision refer to the maximum age(s), as specified by an operator or the Authority, beyond which pilot privileges are curtailed or cancelled. Such curtailment or cancellation of privileges is generally associated with flight crew member position and/or flight crew composition. The specifications in item ii) refer to operators that are subject to laws or regulations of the State that preclude the specification of maximum age limits for flight crew members to exercise the full privileges of their pilot license in operations for the operator. It also refers to operators that are subject to laws or regulations of the state that prescribe a different maximum age limit from 65 years. The specifications of this provision may be satisfied by an operator's process for tracking pilot flight crew member age, if age requirements or limits are specified by the operator or Authority. Such tracking might be necessary to conform to State requirements when a pilot crew member changes position or reaches a mandatory age limit.

FLT 3.3.7

The Operator shall have a process to ensure flight crew member recency-of-experience requirements are satisfied as follows: - A pilot does not act as PIC or SIC of an aircraft unless either: - On the same type or variant of aircraft within the preceding 90 days (120 days if under the supervision of an instructor or evaluator), that pilot has operated the flight controls during at least three takeoffs and landings in the aircraft type or in a flight simulator approved for the purpose by the appropriate authority, or - On the same type or variant of aircraft within a time period acceptable to the State and applicable authorities, that pilot has operated the flight controls during the number of takeoffs and landings in the aircraft type or in a flight simulator approved for the purpose by the appropriate authority, necessary to conform to a defined recency of experience schedule approved or accepted by the State and applicable authorities. - A pilot does not act in the capacity of a cruise relief pilot unless, within the preceding 90 days, that pilot has either: - Operated as PIC, SIC or cruise relief pilot on the same type or variant of aircraft, or - Completed flying skill refresher training to include normal, abnormal and emergency procedures specific to cruise flight on the same type of aircraft or in a flight simulator approved for the purpose, and has practiced approach and landing procedures, where the approach and landing procedure practice may be performed as the PM. - flight engineer does not perform duties in an aircraft unless either: - Within the preceding 6 months, that individual has had at least 50 hours of flight time as a flight engineer on that aircraft type aircraft, or - Within the preceding 90 days, that individual has operated as a flight engineer on board that aircraft type or in a simulator of the aircraft type. - A flight navigator or radio operator does not perform duties in an aircraft unless recency-of experience requirements of the Operator and the State have been satisfied. - If a flight crew member does not satisfy recency-of-experience requirements in accordance with i), ii), iii) or iv), such flight crew member completes re-qualification in accordance with the Operator's training and evaluation program. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed tracking/scheduling processes that prevent flight crew members from flight duty assignment unless recency-of-experience qualification requirements are met. ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined guidance/procedures (focus: definition of recency-of-experience qualification requirements). ☐ Examined selected flight crew training/qualification records (focus: satisfaction of recency-of experience qualification requirements). ☐ Observed flight crew scheduling operations (focus: scheduling tracks/accounts for flight crew member recency-of-experience qualification requirements). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The specification in item i) requires the pilots to operate the flight controls: PM duties do not satisfy recency-of-experience requirements for this specification. The specifications in item (i) also ensure that newly qualifying pilots have the necessary experience to operate as a required crew member in the line training qualification program. The process to ensure such pilots meet recency-of-experience requirements may be integral to the line qualification program in accordance with FLT 2.3.1. The term Pilot Monitoring (PM) has the same meaning as the term Pilot Not Flying (PNF) for the purpose of applying the specifications of this provision. According to a defined schedule in order to establish an equivalent level of recency experience. Such schedule would not have to adhere exactly to the specification in item i) a) of this provision if the level of recent experience is acceptable to the State and applicable authorities, and the PIC or SIC, as applicable, is required to operate the flight controls in order to satisfy recency-of-experience requirements. Item v) specifies that a flight crew member whose recency has lapsed for any reason becomes unqualified and must be re-qualified by the operator. The requalification program for such a flight crew member need not specify the same number of takeoffs and landings as the recency requirements. Applicable authorities include those authorities that have jurisdiction over international operations conducted by an operator over the high seas or the territory of a state that is other than the State of the Operator. The term “abnormal” is used to describe a condition or situation (e.g. abnormal airframe vibration, abnormal landing configuration). The terms “normal” and “non-normal/emergency” typically refer to AOM checklists, procedures and/or maneuvers. The term “non-normal” includes AOM emergency checklists and/or procedures (i.e. an emergency procedure is a subset of non-normal). The terms can also be used to describe an event, situation or operation that would be addressed by normal or non-normal/emergency procedures or checklists. When used in this manner, the terms may be separated by forward slash marks (e.g. normal/non-normal/emergency). The term “emergency” used alone refers to declarations and non-AOM procedures.

FLT 3.3.9

The Operator shall have an airport qualification process that ensures a PIC has made an actual approach and landing at each airport within the Operator's route system accompanied by a pilot, either as a crew member or flight deck observer, that is qualified for that airport, unless: - The approach to the airport is not over difficult terrain and the instrument approach procedures and aids available are similar to those with which the pilot is familiar, and the normal operating minima are adjusted by the addition of a margin of safety that is approved or accepted by the State, or there is reasonable certainty that approach and landing can be made in visual meteorological conditions (VMC), or - The descent from the initial approach altitude can be made by day in VMC, or - The Operator has qualified the PIC for operations into the airport by means a pictorial representation that is approved or accepted the Authority, or - The airport is adjacent to another airport into which the PIC is currently qualified to operate. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed tracking/scheduling/pairing processes for ensuring PICs will meet qualification requirements for airports/areas/routes to be used in operations. ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined OM guidance/procedures (focus: definition of crew member qualification criteria for operations into airports/areas/routes used in operations). ☐ Observed flight crew scheduling operations (focus: scheduling and crew pairing accounts for PIC qualification for operations into airports of intended landing). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The specification in item (i) may be satisfied by a process, approved or accepted by the State, that: - Identifies instrument approach procedures that require the application of margins to operating minima; - Specifies the operating margin to be applied. The specification in item (iii) may be satisfied by any pictorial representation approved or accepted for the purpose by the Authority, such as an instrument approach plate or chart. Refer to FLT 2.3.1 and associated Guidance for additional specifications and information that addresses training for operations associated with special areas, routes, route segments and special airports.

 FLT 3.3.10

The Operator shall have a process to ensure a pilot is not used as a PIC in operations that require the application of special skills or knowledge to operate into special airports, as designated by the State or by the Operator, unless, within the preceding 12 months, that pilot has either: - Made at least one trip as a pilot flight crew member, line check airman or observer on the flight deck on a route in close proximity and over similar terrain within the specified area(s), on the specified route and/or into the special airport, as applicable, or - Completed training and an evaluation in the special skills and/or knowledge required to qualify or requalify for such operations. The content of training shall ensure the PIC has adequate knowledge of the elements specified in Table 2.5 as applicable to the areas, routes, route segments and special airports of intended operation, or - The Operator has qualified the PIC for operations into the special airport by means a pictorial representation that is approved or accepted the Authority. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed tracking/scheduling processes that prevent PICs from flight duty assignment into airports/areas and on routes/route segments that require special skills/knowledge, unless qualification requirements have been satisfied. ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined OM guidance/procedures (focus: definition of PIC qualification criteria for operations into airports/areas/routes that require special knowledge/skills). ☐ Observed flight crew scheduling operations (focus: scheduling tracks/accounts for PIC qualification for routes/airports that require special knowledge/skills). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Special airport re-qualification (if applicable) could take the form of pictorial review, simulator training, line check airmen briefing or operation into the airport accompanied by a line check airman or other qualified airman and could include exemptions for VFR operations. The intent of this provision is to ensure the PIC has a level of knowledge of terrain, minimum safe altitudes, seasonal meteorological conditions, communication and air traffic facilities, services and procedures, search and rescue services and navigational facilities and procedures, required for safe operations. Refer to FLT 2.4.1 and associated Guidance for additional specifications and information that addresses training for operations associated with special airports.

### 3.4 Flight Crew Scheduling

FLT 3.4.1

The Operator shall have a means to ensure flight crew members are qualified and current prior to accepting and/or being assigned to duty. Such means shall consist of: - A requirement that prohibits flight crew members from operating an aircraft if not qualified for duty in accordance with requirements contained in Table 2.3; - A scheduling process that ensures flight crew members, prior to being assigned to duty, are qualified and current in accordance with the applicable flight crew qualification requirements contained in Table 2.3 and, if applicable, additional requirements of the State. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed tracking/scheduling processes that prevent flight crew members from flight duty assignment unless currently qualified in accordance with Table 2.3 or other applicable requirements of the State. ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined process for determining additional flight crew qualification requirements of the State. ☐ Examined selected flight crew duty assignment records (focus: satisfaction of applicable qualification requirements). ☐ Observed flight crew scheduling operations (focus: scheduling requires flight crew member qualification in accordance with Table 2.3 and requirements of State). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The intent of this provision is to ensure flight crew member requirements and related scheduling processes preclude operation of an aircraft by a flight crew member that is not qualified and current in accordance with the specifications of the provision.

FLT 3.4.3

The Operator shall have a methodology for the purpose of managing fatigue-related safety risks to ensure fatigue occurring in one flight, successive flights or accumulated over a period of time does not impair a flight crew member's alertness and ability to safely operate an aircraft or perform safety-related duties. Such methodology shall consist of flight time, flight duty period, duty period and rest period limitations that are in accordance with the applicable prescriptive fatigue management regulations of the State.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed requirements/methodology for flight crew fatigue management. ☐ Identified/Assessed tracking/scheduling processes (focus: processes take into account flight time/flight duty period/duty period/rest period limitations in the duty assignment of flight crew members). ☐ Interviewed responsible manager(s) in flight operations. ☐ Interviewed selected scheduling personnel. ☐ Examined selected flight crew duty assignment records (focus: examples of application of flight crew fatigue management limitations/mitigations). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The intent of this provision is to ensure an operator establishes a methodology for the management of crew member fatigue in a manner that: - Is based upon scientific principles and knowledge; - Is consistent with the prescriptive fatigue management; - Precludes fatigue from endangering safety of the flight.

### 3.6 Route and Airport Planning

FLT 3.6.4

If the Operator is authorized for LVO, the Operator shall have guidance that enables the flight crew to determine Runway Visual Range (RVR) requirements for runways of intended use, to include, as a minimum: - Requirement for the availability of RVR reporting in order for CAT II and CAT III approach and landing operations to be authorized; - Required minimum RVR values for takeoff and authorized approaches; - Required minimum RVR values that consider inoperative approach/runway lighting, inoperative transmissometers or inadequate visual reference. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed OM guidance that specifies takeoff/landing runway visual range (RVR) requirements/associated limitations for runways of intended use (focus: availability to flight crew; instructions for use of information in operations) ☐ Interviewed responsible manager(s) in flight operations. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The means of RVR measurement typically varies depending on the State. The specification in item iii) may be satisfied by a corrections table or manual corrections for inoperative equipment applied to published minima.

### 3.7 Fuel, Weight/Mass and Balance, Flight Plans

FLT 3.7.2

The Operator shall delegate the authority to the PIC to ensure: - A flight is not commenced unless the usable fuel required in accordance with DSP 4.3.1 is on board the aircraft and is sufficient to complete the planned flight safely; - If fuel is consumed during a flight for purposes other than originally intended during pre-flight planning, such flight is not continued without a re-analysis and, if applicable, adjustment of the planned operation to ensure sufficient fuel remains to complete the flight safely. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed OM requirement for PIC to ensure required safe usable fuel on board prior to flight (focus: delegation of authority to PIC; instructions for determination of safe usable fuel). ☐ Interviewed responsible manager(s) in flight operations. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Refer to the IRM for the definition of Discretionary Fuel. The intent of this provision is for the PIC to have the authority to ensure sufficient fuel is on board the aircraft to commence or continue the planned flight safely, and to be able to authorize the loading of Discretionary Fuel if such fuel is required for the safe conduct of the flight and will not cause operating limits to be exceeded In a shared system of operational control, the PIC and the Flight Dispatcher/Flight Operations Officer share the responsibility to ensure operating limitations are not exceeded and sufficient fuel is on board to commence or continue the planned flight safely The extent of the re-analysis or adjustment specified in item ii) is commensurate with the scope and complexity of the planned operation.

### 3.8 Aircraft Preflight and Airworthiness

FLT 3.8.1

The Operator shall have guidance and procedures that describe flight crew duties and responsibilities for the use and/or application of the ATL, MEL and CDL. Such guidance and procedures shall be included in the OM or in other documents that are available to the flight crew during flight preparation and accessible to the flight crew during flight, and shall address, as a minimum, PIC responsibilities for: - Determining the airworthiness status of the aircraft; - Ensuring, for each flight, a description of known or suspected defects that affect the operation of the aircraft is recorded in the ATL; - Precluding a flight from departing until any defect affecting airworthiness is processed in accordance with the MEL/CDL; - Ensuring the aircraft is operated in accordance with any applicable MEL/CDL Operational Procedure. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed OM guidance/procedures for flight crew use of ATL/MEL/CDL (focus: availability/accessibility to flight crew prior to/during flight; instructions for use of ATL/MEL/CDL, application of limitations). ☐ Interviewed responsible manager(s) in flight operations. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance RThe intent of this provision is for the operator to have guidance that ensures the proper use and application of the ATL, MEL and CDL. Such guidance typically addresses: - Flight crew responsibilities related to a review of the ATL and the application of the MEL/CDL; - Instructions for when to reference the MEL/CDL regarding a malfunctioning system or component; - Instructions for the completion of log book entries that ensure defects are properly recorded for the purpose of remediation and processing in accordance with the MEL/CDL, as applicable; - If applicable, the fault identification codes, trouble codes or other entries that ensure defects are appropriately identified, categorized and tracked for the purposes of remediation and/or to identify chronic or repetitive unserviceable items; - Flight crew responsibilities related to the repetitive system or component checks that are required to conform to the MEL (e.g. verifying a redundant system is operable in the case of a single system failure); - Any additional guidance necessary to ensure the ATL, MEL and CDL are used and applied in accordance with operator requirements. The specifications of this provision also apply to equivalents for the MEL and CDL.

FLT 3.8.10

If the Operator transports passengers and/or supernumeraries without cabin crew, the Operator shall have procedures to ensure, prior to departure of a flight, passengers and/or supernumeraries, as applicable, have been briefed and are familiar with the location and use of safety equipment, to include: - Seat belts; - Emergency exits; - Life jackets (individual flotation devices), if required - Lifesaving rafts, if required - Oxygen masks; - Emergency equipment for collective use. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed OM guidance/procedures for preflight briefing for passengers/ supernumeraries; orientation as to location/use of safety equipment (focus: instructions of conduct of briefing; definition of safety equipment to be addressed/included). ☐ Interviewed responsible manager(s) in flight operations. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The briefing related to the specification in item ii) also typically addresses any applicable requirements and restrictions for personnel seated adjacent to cabin emergency exits.

### 3.9 Ground Handling

FLT 3.9.1

If the Operator conducts passenger flights with or without cabin crew, the Operator shall have a procedure to ensure verification that: - Passenger and crew baggage in the passenger cabin is securely stowed; - If applicable, cargo packages and/or passenger items being transported in passenger seats are properly secured. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified procedure for flight crew or cabin crew to verify cabin security (focus: baggage and cargo packages/passenger items are stowed or properly secured). ☐ Interviewed responsible manager(s) in flight operations. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The intent of this provision is for an operator to have a procedure for verification by the flight crew or cabin crew that all baggage and, if applicable, cargo packages and/or passenger items being transported in passenger seats are stowed or properly secured. Some operators might transport smaller cargo packages (e.g. mail, COMAT items) secured in cabin passenger seats. Some operators might transport certain passenger items secured in cabin passenger seats. These types of items are typically large, valuable or fragile articles belonging to passengers that are not conducive to transport as checked baggage or appropriate for stowage in overhead bins/lockers (e.g. large musical instruments, certain electronic equipment, prominent trophies, works of art). Such items might thus be secured and carried in a dedicated cabin passenger seat (which might be purchased by the passenger-owner for the purpose of transporting the item).

FLT 3.9.6

If the Operator conducts flights from any airport when conditions are conducive to ground aircraft icing, the Operator shall have de-/anti-Icing policies and procedures published in the OM or in other documents that are available to the flight crew during flight preparation and accessible to the flight crew during flight. Such policies and procedures shall address any flight crew duties and responsibilities related to de-/anti-Icing and include: - Holdover Time tables; - A requirement for a member of the flight crew or qualified ground personnel to perform a visual check of the wings before takeoff, if any contamination is suspected; - A requirement that takeoff will not commence unless the critical surfaces are clear of any deposits that might adversely affect the performance and/or controllability of the aircraft; - A statement that delegates authority to the PIC to order De-/Anti-icing whenever deemed necessary. (GM) Note: The specifications of this provision are also applicable for flights such as ferry, maintenance check/test and training.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed OM policy/procedures for aircraft de-/anti-icing of aircraft (focus: availability/accessibility to flight crew prior to/during flight; description of flight crew authority/duties/responsibilities; statement that requires critical surfaces to be clear of ice prior to takeoff). ☐ Interviewed responsible manager(s) in flight operations. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Refer to the IRM for the definitions of De-/Anti-icing Program and Holdover Time. The intent of this provision is to ensure flight crew members adhere to the clean aircraft concept prior to takeoff anytime there is a potential for the accretion of ice on aircraft critical surfaces during ground operations. Refer to Guidance associated with GRH 4.2.1 located in ISM Section 6 for specifications and associated guidance related to the establishment and maintenance of a De-/Anti-icing Program. Qualified ground personnel specified in item (ii) are typically used to perform a visual wing check in instances when the wings are not visible to the flight crew from the interior of the aircraft (e.g., cargo aircraft operations). The surfaces specified in item (iii) include: wings, flight controls, engine inlets, fuselage surfaces in front of engines or other areas defined in the AOM.

### 3.10 Airspace Rules

FLT 3.10.1

The Operator should require all commercial and non-commercial flights to be conducted under an IFR Flight Plan and in accordance with an IFR clearance. (GM) Note: Non-commercial flights in this ISARP does not mean ferry flights, test/maintenance check flights or training flights.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed OM requirement for all flights to be conducted under IFR flight plan/in accordance with IFR clearance (focus: flight crew filing of IFR flight plan, acceptance of/adherence to IFR clearance). ☐ Interviewed responsible manager(s) in flight operations. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Refer to the IRM for the definition of Instrument Flight Rules (IFR) and Visual Flight Rules (VFR). The intent of this provision is for an operator to file an IFR flight plan with the appropriate ATS unit and obtain an IFR clearance in order to ensure its flights are afforded all of the air traffic services applicable to aircraft operating under IFR within controlled airspace. Such services typically include: - Maintenance of minimum separation standards; - Traffic advisory information; - Terrain or obstruction alerting; - Low altitude alerting; - Strategic route planning; - Automatic flight plan closure at airports with functioning control towers. The specifications of this provision do not preclude an operator from: - Operating certain portions of a commercial or non-commercial flight under VFR (visual flight rules); - Where possible, identifying portions of flights to be flown under VFR on the ATS flight plan (in lieu of filing a purely IFR Flight Plan); - Operating flight such as, maintenance, repositioning, trainig under VFR.

FLT 3.10.4

The Operator shall have guidance that addresses the use of standard radio phraseology when communicating with ATC, the acceptance and readback of ATC clearances and, when necessary, the clarification of such clearances to ensure understanding. Such guidance shall include, as a minimum: - A requirement for the use of the call sign; - A requirement for at least two flight crew members to monitor and confirm clearances to ensure a mutual (flight crew) understanding of accepted clearances under circumstances, as determined by the operator or flight crew, when a missed or misunderstood clearance could pose a safety risk to the flight; - A requirement to clarify clearances with ATC whenever any flight crew member is in doubt regarding the clearance or instruction received. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed OM requirement/guidance for standard radio phraseology in communication with ATC (focus: instructions/procedures for flight crew communications with ATC; definition/use of standard phraseology). ☐ Interviewed responsible manager(s) in flight operations. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The intent of this provision is for an operator to have policies and procedures that ensure: - The use of standard radio phraseology; - ATC clearances are clearly understood during times of increased operational risk. The specification in item ii) refers to situations when a missed or misunderstood clearance could pose a safety risk to the flight (e.g. inadequate terrain clearance, runway incursion, loss of separation). ATC clearances that have the potential to pose such safety risks, if misunderstood by the flight crew, typically include the following: - Heading, altitude/flight level, assigned route/waypoint changes; - Frequency changes during critical phases of flight; - Instructions for any operation on or near a runway.

FLT 3.10.6

The Operator shall have procedures and/or limitations that address operations into and out of uncontrolled airspace and/or airports, to include, if applicable, a prohibition if such operations are not permitted in accordance with restrictions of the AOC or equivalent documents. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed OM procedures/limitations for operations into/out of uncontrolled airspace/airports (focus: flight crew actions/responsibilities for airspace/airport operations with no ATC). ☐ Interviewed responsible manager(s) in flight operations. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance An uncontrolled airport is an airport without an operating control tower. A controlled airport is an airport with a manned and operating control tower surrounded by controlled airspace. Procedures and limitations typically include aircraft position radio broadcast procedures, VFR weather requirements and the ability to receive ATC clearance within a specified time/distance from the departure airport.

### 3.11 In-flight Operations

FLT 3.11.4

The Operator shall ensure minimum flight altitude information applicable to all phases of a flight, including guidance that specifies when descent below any applicable prescribed minimum altitude is permissible, is made available to the flight crew along with instructions for the use of such information. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed OM guidance that specifies when descent below applicable prescribed minimum altitude is permissible (focus: availability of minimum altitude information to flight crew during flight; instructions/procedures for adherence to/descent below minimum altitudes all phases of flight). ☐ Interviewed responsible manager(s) in flight operations. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Minimum prescribed safety altitudes typically include: - Minimum Safety Altitude (MSA); - Minimum Descent Altitude/Height (MDA/H); - Minimum En route Altitude (MEA); - Minimum Obstruction Clearance Altitude (MOCA); - Minimum Off-Route Altitude (MORA); - Minimum Vectoring Altitude (MVA); - Any other minimum altitudes prescribed by the Authority. The specification in item ii) refers to situations when a missed or misunderstood clearance could pose a safety risk to the flight (e.g. inadequate terrain clearance, runway incursion, loss of separation). ATC clearances that have the potential to pose such safety risks, if misunderstood by the flight crew, typically include the following: - Heading, altitude/flight level, assigned route/waypoint changes; - Frequency changes during critical phases of flight; - Instructions for any operation on or near a runway.

FLT 3.11.6

If the Operator conducts single-pilot operations, the Operator shall ensure the operation is not conducted under IFR or at night by a single pilot unless: - the flight manual does not require a flight crew of more than one; - the airplane is propeller-driven; - the maximum approved passenger seating configuration of the aereplane is not more than nine; - the maximum certificated take-off mass does not exceed 5,700 kg; - the pilot-in-command has satisfied the requirements of experience, training, checking, and recency according to the requirments of the Authority

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed OM requirement for single-pilot operations ☐ Interviewed responsible manager(s) in flight operations. ☐ Other Actions (Specify) Click or tap here to enter text.

FLT 3.11.7

The Operator shall have a policy and/or procedures that require the flight crew to monitor fuel during flight to ensure a fuel quantity upon landing that is not less than final reserve fuel. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed OM requirement/guidance for monitoring en route fuel to ensure landing with not less than final reserve fuel (focus: instructions/procedure for flight crew fuel monitoring to ensure landing with final reserve fuel as specified in DSP 4.3.12). ☐ Interviewed responsible manager(s) in flight operations. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Refer to FLT 3.14.16 and FLT 3.14.17 for actions to be taken by the PIC in the event the final reserve minimum fuel quantity specified in DSP 4.3.12 cannot be protected in flight and preserved upon landing.

FLT 3.11.18

If the Operator conducts two pilot operations, the Operator shall have policies and guidance that define and address the division of duties related to the performance and prioritization of flight crew member operational tasks, to include, as a minimum: - A requirement and procedures for the use of checklists prior to, during and after all phases of flight, and in abnormal and emergency situations; - PM/PF duties for all phases of flight, to include normal, abnormal and emergency situations; - PM/PF actions during manual and automatic flight; - Flight and cabin crew duties during situations that require coordination, to include, as a minimum, emergency evacuation, medical emergency and incapacitated flight crew member. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed OM policy/requirement/guidance for sharing/prioritization in performance of flight crew operational tasks (focus: guidance that addresses use of checklists; defines PF/PM duties/task sharing; defines flight/cabin crew duties/task sharing). ☐ Interviewed responsible manager(s) in flight operations. ☐ Other Action (Specify) Click or tap here to enter text.

Guidance The intent of this provision is to ensure flight crew duties are defined and appropriately divided, and that compliance with all applicable checklists contained in the AOM, MEL and CDL occurs in accordance with the operator's task sharing policy. Elements of task sharing are described in the following table. Task sharing is observed during most phases of flight and addresses areas such as: - Philosophy for the use of checklists; - Performance calculations; - Automated flight procedures for flight crew; - Manual flight procedures for flight crew; - Flight crew briefings; - Administrative duties at the appropriate times (such as top of descent and prior to commencing approach). Task sharing is applicable during emergency situations such as: - Rejected takeoff; - Engine failure or fire at V1; - TCAS/ACAS resolution advisory (RA) if applicable; - GPWS Alert if applicable; - Emergency descent. Task sharing is applicable during emergency situations that require coordination with the cabin crew such as: - Emergency evacuation; - Medical emergency; - Flight crew member incapacitation.. The term Pilot Monitoring (PM) has the same meaning as the term Pilot Not Flying (PNF) for the purpose of applying the specifications of this provision. The term “abnormal” is used to describe a condition or situation (e.g. abnormal airframe vibration, abnormal landing configuration). The terms “normal” and “non-normal/emergency” typically refer to AOM checklists, procedures and/or maneuvers. The term “non-normal” includes AOM emergency checklists and/or procedures (i.e. an emergency procedure is a subset of non-normal). The terms can also be used to describe an event, situation or operation that would be addressed by normal or non-normal/emergency procedures or checklists. When used in this manner, the terms may be separated by forward slash marks (e.g. normal/non-normal/emergency). The term “emergency” used alone refers to declarations and non-AOM procedures.

FLT 3.11.46

The Operator shall provide, and require compliance with, operating limitations, as defined by the original equipment manufacturer (OEM) and established by the State of Registry for each aircraft type used in operations.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed OM provision of/requirement for compliance with operating limitations as defined by OEM (focus: guidance/procedures for flight crew compliance with operating limitations). ☐ Interviewed responsible manager(s) in flight operations. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Refer to FLT 3.14.16 and FLT 3.14.17 for actions to be taken by the PIC in the event the final reserve minimum fuel quantity specified in DSP 4.3.12 cannot be protected in flight and preserved upon landing.

FLT 3.11.59

The Operator shall have a stabilized approach policy with associated guidance, criteria and procedures to ensure the conduct of stabilized approaches. Such policy shall specify: - A minimum height for stabilization not less than 1000 feet AAL for approaches in IMC or not less than 500 ft. AAL for approaches in IMC as designated by the operator and/or State where a lower stabilization height is operationally required; - A minimum height for stabilization not less than 500 feet AAL for approaches in VMC; - Aircraft configuration requirements specific to each aircraft type (landing gear, wing flaps, speed brakes); - Speed and thrust limitations; - Vertical speed limitations; - Acceptable vertical and lateral displacement from the normal approach path. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed OM policy/guidance/procedures for the conduct of a stabilized approach (focus: flight crew procedures/definition of criteria for stabilized approach). ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined selected output from FDA/FDM/FOQA program (if applicable) (focus: data that indicates status of fleet stabilized approach performance). ☐ Examined relevant safety objectives including SPIs/SPTs (focus: proactive measures in place for identifying and preventing unstabilized approaches). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Refer to the IRM for the definition of Flight Data Analysis (FDA) Program, Stabilized Approach and Stabilization Heights. The intent of this provision is for the operator to implement a stabilized approach policy, as well as have guidance, criteria and procedures that ensure the maintenance of the intended lateral and vertical flight path during visual approaches and/or as depicted in published approach procedures without excessive maneuvering. The parameters to be considered at the 1000 ft. AAL and 500 ft. gates as well as in the definition of a stabilized approach are listed in items iii) through vi) of the provision. The specifications in item (i) permit an operator, in accordance with operational requirements approved or accepted by the Authority, to establish stabilization criteria for heights lower than 1000 ft. AAL, but no lower than 500 ft. AAL (IMC or VMC), for approaches designated by the operator and/or State where: - Lower minimum approach stabilization heights are authorized for turbo-propeller aircraft operations (e.g., 500 feet AAL on VMC/IMC approaches), and/or - Maneuvering at a lower height AAL is required to meet instrument or other charted approach constraints (e.g. RNAV/RNP approaches, circling approaches and charted visual approaches), and/or - Aircraft are required to comply with ATC speed constraints on final approach, and/or - Deviations from selected approach stabilization criteria at a height lower than 1000 feet AAL, but above 500 feet AAL, are operationally required, and the operator can demonstrate pilot adherence to its stabilized approach policy via a continually monitored, managed and active flight data analysis (FDA) program. The criteria used to conform to the specifications in item (vi) also typically address the maneuvering that may be required in accordance with a charted visual or instrument approach procedure.

FLT 3.11.60

The Operator shall have a go-around policy with associated procedures and guidance to ensure, when necessary, flight crews discontinue or go around from an approach in accordance with criteria established by the Operator. Such policy, procedures and guidance shall, as a minimum, address or define: - Management support for flight crew go-around decision making; - Criteria that require flight crews to discontinue or go around from an approach or a landing (prior to the selection of reverse thrust, if applicable); - The go-around maneuver; - Duties and responsibilities of the PF and PM. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed OM policy/requirements for execution of a missed approach/go-around when approach not stabilized in accordance with established criteria (focus: flight crew guidance/procedures for execution of a missed approach/go-around). ☐ Interviewed responsible manager(s) in flight operations. ☐ Examined selected output from FDA/FDM/FOQA program (if applicable) (focus: data that indicates fleet status of missed approach/go-around from unstabilized approach). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The specifications of this provision are directly related to the prevention of approach and landing accidents (ALAs) such as CFIT and runway excursions. The intent of this provision is to reduce the risk of ALAs by ensuring the flight crew will always discontinue or go around from an approach or landing (prior to the selection of reverse thrust) when a safe landing cannot be assured or a go-around is otherwise required. The specification in item (i) is intended to foster a culture that supports flight crew go-around decision making. It is typically expressed by senior management in a manner that: - Promotes the go-around as a normal procedure; - Encourages go-around preparedness and considers the risk of the go-around maneuver itself; - Empowers the PM (or the SIC) to call for a go-around at any time during approach and landing until the selection of reverse thrust; - Ensures that go-around decision making does not affect the PIC’s emergency authority in the event of (impending) abnormal or emergency situations; - Does not inhibit flight crew reporting of go-around related events. The criteria referred to in item (ii), which would require a go-around or discontinuation of an approach, typically include: - The visibility or ceiling is below the minimum required for the type of approach at the specified gates (e.g. outer marker, 1,000’ AAL or at minimums). - The appropriate visual references are not obtained or are lost at or below MDA (or minimum descent height) or DA (or decision height) and through flare and touchdown by either pilot. - Prior to touchdown the wind is above the operational or pre-determined wind limit, or the runway status is below the limit determined by the flight crew’s landing performance assessment. - The criteria for a stable approach are not met at the relevant approach gate(s) or can no longer be maintained until touchdown. - Technical defects or failures occur during approach that might inhibit a safe continuation of approach, landing or go-around. - Doubts by either pilot about the aircraft’s geographic or spatial position. - Confusion by either pilot about the use or behavior of the automation. - It is foreseeable that the go-around routing and path will not be sufficiently clear of adverse weather or restricting traffic. - If instructed by ATC. - If required for type-specific reasons as outlined in the respective AOM, - If required by special considerations associated with a CAT II/III operation. The specification in item (iii) refers to the aircraft type-specific maneuver(s) for discontinuing a visual approach, an instrument approach or a landing prior to the selection of reverse thrust (i.e. rejected landing). The specification in item (iv) typically addresses: - Timely and effective PF briefings. - PM stabilized approach criteria deviation callouts and compliance checks: - PF and/or PM go-around callouts and subsequent execution of the go-around maneuver. - PF/PM go-around-related memory items. - PM actions in the event of (subtle) PF incapacitation or delayed response to a go-around callout. - PF/PM actions in the event of destabilization below stabilization height including PM monitoring for possible excessive deviations from flight path, speed, vertical speed, pitch or bank during the approach, during the transition from approach to landing and during flare and touchdown. - As applicable, the role of additional flight crew members on the flight deck (e.g., augmented crew members). To support SRM activities an operator would typically: - Include and monitor aircraft parameters related to CFIT and runway excursions in their flight data analysis (FDA) program in accordance with provisions in ORG sub-section 3.3. - Monitor go-around policy compliance through their FDA program and establish go-around safety performance indicators (SPIs). In addition to monitoring go-arounds, aircraft operators would also monitor discontinued approaches. - Include unstable approaches followed by a landing as a reporting event by the flight crew. - Minimize the need for the flight crew to report a go-around due to an unstable approach unless there is another significant event associated with the go-around (e.g. flap overspeed). An operator, in accordance with requirements of the Authority and consistent with OEM guidance, typically develops a go-around policy, guidance, criteria and procedures based on one or more of the following source references: - Global Action Plan for the Prevention of Runway Excursions Coordinated by EUROCONTROL and the Flight Safety Foundation – January 2021; - Flight Safety Foundation Go-Around Decision-Making and Execution Project Final Report March 2017; - Flight Safety Foundation Reducing the Risk of Runway Excursions – Report of the Runway Safety Initiative – May 2009; - BEA Study on Aeroplane State Awareness during Go-Around - August 2013 - Any equivalent reference document approved or accepted by the Authority for the development of flight crew guidance related to the establishment of go-around policy and the prevention of unstable approaches and runway excursions.

FLT 3.11.62

The Operator shall have a policy and procedures to ensure the flight crew will not continue an instrument approach to land at any airport beyond a point at which the limits of the operating minima specified for the approach in use would be infringed. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed OM policy/procedures that address continuation of an instrument approach to landing beyond limits of specified operating minima (focus: flight crew requirement/procedures for maintaining adherence to operating minima). ☐ Interviewed responsible manager(s) in flight operations. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The intent of this provision is to ensure a transition to the missed approach is initiated at a designated point or height AAL that prevents infringing on the operating minima specified for the approach. The standard specifies actions required from the flight crew when reaching the limit of the approach, (i.e. when reaching the DA(H) or MDA(H) or equivalent).

FLT 3.11.68

The Operator shall have a policy and procedures to ensure an approach is not continued below 300 m (1 000 ft) AAL unless the PIC is satisfied that, with the runway surface condition information available, the aircraft landing performance assessment indicates that a safe landing can be made. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed policy/procedures for discontinuing an approach if the runway surface condition would prevent a safe landing. ☐ Interviewed responsible manager(s) in flight operations. ☐ Interviewed flight crew members in flight operations. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance This specifications of this provision are directly related to the prevention of runway excursions.

### 3.12 Flight Deck Policy and Procedures

FLT 3.12.1

The Operator shall have a corrective lenses policy that addresses the need for flight crew members, who are required to use corrective lenses, to have a spare set of corrective lenses readily available. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed policy/requirement for flight crew members that require use of corrective lenses to have a spare set readily available (focus: flight crew requirement for availability of spare corrective lenses). ☐ Interviewed responsible manager(s) in flight operations. ☐ Interviewed flight crew members in flight operations. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Corrective lens requirements are typically listed on a medical certificate or license issued by the State.

FLT 3.12.2

The Operator shall have a policy that requires flight crew members to keep their seat belts fastened when at their assigned stations and: - Those flight crew members occupying a pilot's seat to keep their safety harnesses (shoulder straps and seat belts) fastened during the takeoff and landing phases of flight; - Other flight crew members to keep their safety harnesses fastened during the takeoff and landing phases of flight, unless the shoulder straps interfere with the performance of duties, in which case the shoulder straps may be unfastened but the seat belts shall remain fastened.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed OM policy/requirements for flight crew use of seat belts/safety harnesses when at their assigned stations (focus: definition of requirements for flight crew members to have seat belts/safety harness fastened). ☐ Interviewed responsible manager(s) in flight operations. ☐ Interviewed flight crew members in flight operations. ☐ Other Actions (Specify) Click or tap here to enter text.

FLT 3.12.4

If applicable, the Operator shall have a policy and procedures to ensure flight crew members are only permitted to leave their duty stations during flight in the performance of duties or to meet physiological needs and shall always remain on their stations during take-off and landing. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed OM policy/procedures that address flight crew members leaving duty stations during flight (focus: requirement that flight crew member may leave duty station only for performance of duties/physiological needs). ☐ Interviewed responsible manager(s) in flight operations. ☐ Interviewed flight crew members in flight operations. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The specifications of this provision do not apply to crew changes that occur in conjunction with relief and/or augmented crews.

### 3.14 Non-Normal/Abnormal and Emergency Operations

FLT 3.14.1

If the Operator utilizes single-engine aircraft, the Operator shall ensure that all single-engine aeroplanes are operated in conditions of weather and light, and over routes and diversions therefrom, that allow for a safe forced landing/ditching to be executed in the event of engine failure.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed OM procedures of forced landing in the event of engine failure. ☐ Interviewed responsible manager(s) in flight operations. ☐ Interviewed flight crew members in flight operations. ☐ Other Actions (Specify) Click or tap here to enter text.

FLT 3.14.2

The Operator shall have a policy that prohibits the in-flight simulation of emergencies while passengers and/or cargo are being transported on board the aircraft.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed OM policy that prohibits in-flight simulated emergencies with passengers/cargo on board the aircraft. ☐ Interviewed responsible manager(s) in flight operations. ☐ Interviewed flight crew members in flight operations. ☐ Examined training/qualification program for instructors/evaluators/line check airmen (focus: prohibition of in-flight simulated emergencies with passengers/cargo on board the aircraft). ☐ Other Actions (Specify) Click or tap here to enter text.

FLT 3.14.16

The Operator shall have an in-flight fuel management policy that requires the PIC to advise ATC of a minimum fuel state: - When, having committed to land at a specific airport, the PIC calculates that any change to the existing clearance to that airport may result in landing with less than planned final reserve fuel; - By declaring “MINIMUM FUEL”. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed OM policy/procedures for in-flight fuel management (focus: flight crew procedures for monitoring en route fuel usage/identifying trends; requirement for flight crew to declare minimum fuel when minimum fuel for landing at destination airport might be less than planned final reserve fuel). ☐ Interviewed responsible manager(s) in flight operations. ☐ Interviewed flight crew members in flight operations. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The intent of a “MINIMUM FUEL” declaration is to inform ATC that the flight has committed to land at a specific airport and any change to the existing clearance may result in landing with less than planned final reserve fuel. This is not an emergency situation, but rather an indication that an emergency situation is possible should any additional delay occur. Guidance on in-flight fuel management, including minimum fuel declarations, is contained in the ICAO Flight Planning and Fuel Management Manual (Doc 9976).

FLT 3.14.17

The Operator shall have an in-flight fuel management policy that requires the PIC to declare a situation of fuel emergency: - When the calculated usable fuel predicted to be available upon landing at the nearest airport where a safe landing can be made is less than the planned final reserve fuel; - By declaring “MAYDAY, MAYDAY, MAYDAY, FUEL”. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed OM policy/procedures for in-flight fuel management (focus: flight crew procedures for monitoring en route fuel usage/identifying trends; requirement for flight crew to declare an emergency when minimum fuel for landing at nearest airport is calculated to be less than planned final reserve fuel). ☐ Interviewed responsible manager(s) in flight operations. ☐ Interviewed flight crew members in flight operations. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The intent of this provision is to specify the last procedural step in a series of steps to ensure the safe completion of a flight. The “MAYDAY, MAYDAY, MAYDAY, FUEL” declaration provides the clearest and most urgent expression of an emergency situation brought about by insufficient usable fuel remaining to protect the planned final reserve. It communicates that immediate action must be taken by the PIC and the air traffic control authority to ensure that the aircraft can land as soon as possible. It is used when all opportunities to protect final reserve fuel have been exploited and in the judgment of the PIC, the flight will now land with less than final reserve fuel remaining in the tanks. The word fuel is used as part of the declaration simply to convey the exact nature of the emergency to ATC. Guidance on in-flight fuel management including emergency fuel declarations is contained in the ICAO Flight Planning and Fuel Management Manual (Doc 9976).

### 3.15 Flight Crew Reporting Requirements

FLT 3.15.2

The Operator shall have a policy that requires the PIC to report any hazardous flight condition to the appropriate ATC facility without delay. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed policy for flight crew ATC hazard reporting (focus: flight crew procedures for reporting occurrences that could potentially have adverse effect on safety of flight operations). ☐ Interviewed responsible manager(s) in flight operations. ☐ Interviewed selected flight crew members. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The intent of this provision is to ensure hazards with the potential to pose safety risks to the reporting aircraft or flight operations are appropriately identified and reported to the applicable ATS unit as soon as possible. Such required reports are typically defined by the State or applicable authorities and may include types of hazards as described in the following table. Generic Hazard; Report Description. Meteorological Conditions; Un-forecast or severe weather, icing, wind shear, severe turbulence. Geophysical Events; Volcanic ash observed or encountered. Security Breaches; Air Piracy or other hostile acts that threaten the safety of the aircraft or its passengers. Wildlife; Birds or large animals in the vicinity of the airport or runways. Facilities and Infrastructure; Inadequacy of navigational facilities or undesirable navigational aid performance or other irregularity in navigational or ground facilities. Aircraft Performance; Unable to accept or maintain RVSM and reason (e.g. turbulence, mountain wave, wake turbulence, etc.), loss of navigational capability. Lasers; Illumination activities, events or exposure. Dangerous Goods; Dangerous goods on board the aircraft in the case of an in-flight emergency and for the information of airport authorities.. GPS Anomalies; Locations of GPS interference/jamming. Note: Previously promulgated hazard information (e.g., via NOTAM) would not typically require additional reporting by the PIC.

FLT 3.15.3

The Operator shall have a policy that assigns responsibility to the PIC for notifying the nearest authority, by the quickest available means, of any accident or serious incident resulting in injury, death, or substantial aircraft damage. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed policy for flight crew accident/incident reporting (focus: flight crew responsibility/procedures for reporting accidents/serious incidents to the nearest authority by the quickest available means). ☐ Interviewed responsible manager(s) in flight operations. ☐ Interviewed selected flight crew members. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The intent of this provision is to ensure the appropriate authority in the state where an event occurred and any other organization required by such state are expeditiously informed of any accident or serious incident resulting in injury, death, or substantial aircraft damage. Such authority and organization(s) are typically defined in the applicable Aeronautical Information Publication (AIP) and may refer to one or more entities including but not limited to local law enforcement agencies, emergency service providers, the Civil Aviation Authority (CAA) and related air accident branches, safety bureaus or boards (e.g., NTSB). The PIC, if able, typically reports an applicable event to the operator who then forwards it to the appropriate authority and other relevant organization(s).

FLT 3.15.4

The Operator shall have a policy that assigns responsibility to the PIC for: - Notifying the appropriate local authority without delay in the event of any emergency situation that necessitated action in violation of local regulations and/or procedures; - Submitting, if required by the state of occurrence, a report to the appropriate local authority and also to the Authority of the State of the Operator. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed policy for flight crew emergency action reporting (focus: flight crew responsibility/procedures for reporting to the appropriate authorities any emergency situation that necessitated action in violation of local regulations and/or procedures). ☐ Interviewed responsible manager(s) in flight operations. ☐ Interviewed selected flight crew members. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The intent of item (i) is to ensure the appropriate authority, as specified in local instructions, is notified when an in-flight emergency requires the PIC to deviate from a local rule or procedure to the extent required to meet that emergency. Such notifications are typically made through the appropriate air traffic services (ATS) unit and involve a deviation from an assigned clearance or instrument procedure. The intent of item (ii) is to ensure required occurrence/incident reporting takes place in accordance with local regulations or procedures. This includes reports submitted to the ATS unit concerned for occurrences/incidents specifically related to the provision of air traffic services.

FLT 3.15.5

The Operator shall have a policy that requires the PIC to report the runway braking action special airreport (AIREP) when the runway braking action encountered is not as good as reported. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed OM policy for runway braking action reporting by the flight crew. ☐ Interviewed responsible manager(s) in flight operations. ☐ Interviewed selected flight crew members. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Refer to the IRM for the definition of Air-report (AIREP). Refer to ICAO PANS-ATM (Doc 4444), Chapter 4 and Appendix 1, for reporting instructions and guidance that addresses special air-reports regarding runway braking action and the format for transmitting such reports by voice or data link. Refer to ICAO Circular 355 AN/211 for ATS actions when receiving AIREPs concerning braking action that is not as good as that reported.

## 4 Operations Engineering Specifications

General Guidance Refer to the IRM for the definition of Operations Engineering.

### 4.1 Aircraft Performance

FLT 4.1.1

The Operator shall have a process, performed by Operations Engineering, to determine and maintain guidance, procedures and performance data in the OM, applicable to each aircraft type, for applicable departure, destination and alternate airports. Such guidance and data shall enable the flight crew to determine or compute: - Maximum structural weights (taxi, takeoff, landing); - Takeoff performance (accelerate - stop, close-in obstacles) that also ensures charting accuracy is accounted for, when necessary, in assessing takeoff performance in the event of a critical power unit failing at any point in the takeoff; - Maximum brake energy and minimum cooling time if applicable; - Climb performance (distant obstacles); - Landing performance (minimum landing distance, go-around). (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed OM guidance/procedures/data for flight crew calculation of aircraft performance for taxi/takeoff/climb/landing at departure/destination/alternate airports (focus: performance data provided for all aircraft types; OM contains performance data as specified in standard). ☐ Interviewed responsible manager(s) in flight operations. ☐ Interviewed selected operations engineering personnel (focus: process for development of OM performance information/data). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The specifications in this provision are related to the prevention of CFIT and runway excursions. The intent is to ensure the operator has a process or processes to obtain or determine the specified performance data for use by flight crew. Such process(s) also address the maintenance and publication of guidance, procedures, and performance data in the OM. Data may be tailored for airports of intended use (e.g. runway analysis). The specifications in items ii) and v) may necessitate the inclusion of guidance and/or patterns to be followed in case of engine failure during takeoff, approach and go-around. Tailored data is not always available for emergency alternate airports.

FLT 4.1.2

The Operator shall have a process, performed by Operations Engineering, to determine and maintain guidance, data and procedures in the OM, applicable to each aircraft type, that enable the flight crew to determine and/or compute aircraft performance for all phases of flight. Such guidance and data shall ensure the flight crew considers all relevant factors affecting aircraft performance, to include: - Aircraft weight (mass); - Operating procedures; - Pressure altitude appropriate to the airport elevation; - Temperature; - Wind; - Runway gradient (slope); - Runway surface condition at the expected time of use; - Obstacle data; - NOTAMs (including airport NOTAMs); - As applicable, MEL/CDL information; - Aircraft configuration (wing flap setting); - Anti-ice usage and, when applicable, ice accretion; - As applicable, runway length used for aircraft alignment prior to takeoff; - As applicable, fuel freeze considerations during extended operations. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed OM guidance/procedures/data for flight crew calculation of aircraft performance for all phases of flight (focus: performance data provided for all aircraft types; OM guidance/data incorporates relevant factors/limitations as specified in standard). ☐ Interviewed responsible manager(s) in flight operations. ☐ Interviewed selected operations engineering personnel (focus: process for development of OM performance information/data). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The specifications in this provision are related to the prevention of CFIT, runway excursions and inflight loss of control. The intent is to ensure the operator has a process or processes to obtain or determine the specified performance data for use by flight crew. Such process(s) also address the maintenance and publication of guidance, procedures, and performance data in the OM. The specification in item vii) could be defined by a specific contaminant type/depth (e.g. snow, slush, water, ice) or an equivalent braking action report. The specifications in xiii) refers to a determination of the length of the runway available, taking into account the loss, if any, of runway length due to alignment of the aircraft prior to takeoff. The specifications in xiv) apply to considerations regarding the use of standard fuel freeze temperatures, fuel temperature analysis and en route fuel temperature monitoring for the specific fuels used in operations. Such considerations allow the flight crew to determine the actual fuel freeze temperature during extended operations (e.g. polar operations) in order to prevent in-flight freezing of fuel.

FLT 4.1.3

The Operator shall have a process, performed by Operations Engineering, to determine and maintain guidance, data and procedures in the OM, applicable to each aircraft type, that enable the flight crew to determine and/or compute en route aircraft engine-out performance. Such guidance, data and procedures shall include, as a minimum, aircraft engine-out: - Service ceiling; - Drift down altitudes, as well as specific guidance and procedures that assure terrain clearance along the route to the destination airport or to an en route alternate airport. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed guidance/procedures/data for flight crew calculation of en route aircraft engine-out performance (focus: performance data provided for all aircraft types; OM contains engine-out performance data as specified in standard). ☐ Interviewed responsible manager(s) in flight operations. ☐ Interviewed selected operations engineering personnel (focus: process for development of OM performance information/data). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The intent of this provision is to ensure an operator has a process or processes to obtain or determine the specified performance data for use by flight crew. Such process(s) also address the maintenance and publication of guidance, procedures, and performance data in the OM. The specification in item ii) refers to those areas were adequate terrain clearance cannot be assured at the engine-out service ceiling of the aircraft without following specific guidance and procedures for drift down.

### 4.3 Aircraft Systems and Equipment Specifications

FLT 4.3.1

If applicable, the Operator shall ensure all turbine engine aircraft in its fleet with a maximum certificated takeoff mass in excess of 5,700 kg (12,566 lb), or authorized to carry more than nine passengers, are equipped with a ground proximity warning system (GPWS) that automatically provides a warning to the flight crew when the aircraft is in close proximity to the earth's surface and there is: - Excessive descent rate; - Excessive altitude loss after takeoff or go-around; - Unsafe terrain clearance. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Examined turbine engine aircraft type specifications/system descriptions related to GPWS. ☐ Identified/Examined method(s) used to ensure turbine engine aircraft are equipped with GPWS. ☐ Other Action (Specify) Click or tap here to enter text.

Guidance Refer to the IRM for the definition of Ground Proximity Warning System (GPWS). In addition to the above-specified warnings, it is recommended that the GPWS also provide warnings for the following conditions: - Excessive terrain closure rate; - Unsafe terrain clearance when not in the landing configuration: - Landing gear is not locked down; - Flaps are not in the landing position. - Excessive descent below the instrument glide path.

FLT 4.3.2

If applicable, the Operator should ensure all turbine engine aircraft in its fleet with a maximum certificated takeoff mass of 5,700 kg (12,566 lb) or less and authorized to carry more than five passengers are equipped with a GPWS which has a forward–looking terrain avoidance function. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Examined turbine engine aircraft type specifications/system descriptions related to GPWS. ☐ Identified/Examined method(s) used to ensure turbine engine aircraft are equipped with GPWS. ☐ Other Action (Specify) Click or tap here to enter text.

Guidance Refer to the IRM for the definition of Ground Proximity Warning System (GPWS) and GPWS with a Forward-Looking Terrain Avoidance (FLTA) Function. Different systems are available and acceptable as a GPWS with a forward-looking terrain avoidance (FLTA) function, as specified in this provision. The following guidance is an overview only; it is not to be construed as technical specifications for an acceptable system. A GPWS with a FLTA function could also be known as a predictive terrain awareness and warning system (TAWS), and provides: - A forward-looking capability and terrain clearance floor; - The flight crew, by means of visual and aural signals, and a terrain awareness display, with an alerting time necessary to prevent controlled flight into terrain events. An acceptable system provides a forward-looking capability and terrain clearance floor protection in areas of operations and surrounding airports of intended use. Such systems generally have: - A navigation system that provides accurate aircraft position (e.g. GPS or equivalent); - A means of displaying aircraft and terrain information; - A means of providing visual and aural signals; - A terrain database(s) for all areas of potential operations and surrounding airports of intended use. If an obstacle database is commercially available and obstacle detection/display functionality is installed, an obstacle database for all areas of potential operations.

FLT 4.3.3

If applicable, the Operator shall ensure all piston engine aircraft in its fleet with a maximum certificated takeoff mass in excess of 5,700 kg (12,566 lb), or authorized to carry more than nine passengers, are equipped with a GPWS that automatically provides a warning to the flight crew when the aircraft is in close proximity to the earth's surface and there is: - Excessive descent rate; - Excessive terrain closure rate; - Excessive altitude loss after takeoff or go-around.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed piston engine aircraft type specifications/system descriptions related to GPWS. ☐ Identified/Assessed method(s) used to ensure piston engine aircraft are equipped with GPWS. ☐ Other Actions (Specify) Click or tap here to enter text.

FLT 4.3.4

If applicable, the Operator should ensure all piston engine aircraft in its fleet with a maximum certificated takeoff mass in excess of 5,700 kg (12,566 lb), or authorized to carry more than nine passengers, are equipped with a GPWS which has a forward–looking terrain avoidance function. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Examined piston engine aircraft type specifications/system descriptions related to GPWS ☐ Identified/Examined method(s) used to ensure piston engine aircraft are equipped with GPWS. ☐ Other Action (Specify) Click or tap here to enter text.

Guidance Refer to the IRM for the definition of Ground Proximity Warning System with a Forward-Looking Terrain Avoidance (FLTA) Function.

FLT 4.3.5

If applicable, the Operator shall ensure all turbine engine aircraft in its fleet with a maximum certificated takeoff mass in excess of 5,700 kg (12,566 lb), or authorized to carry more than nine passengers, are equipped with a ground proximity warning system (GPWS) which has a forward–looking terrain avoidance function.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Examined turbine engine aircraft type specifications/system descriptions related to GPWS ☐ Identified/Examined method(s) used to ensure turbine engine aircraft are equipped with GPWS. ☐ Other Action (Specify) Click or tap here to enter text.

Guidance Refer to the IRM for the definition of Ground Proximity Warning System (GPWS) and GPWS with a Forward-Looking Terrain Avoidance (FLTA) Function. Different systems are available and acceptable as a GPWS with a forward-looking terrain avoidance (FLTA) function, as specified in this provision. The following guidance is an overview only; it is not to be construed as technical specifications for an acceptable system. A GPWS with a FLTA function could also be known as a predictive terrain awareness and warning system (TAWS), and provides: - A forward-looking capability and terrain clearance floor; - The flight crew, by means of visual and aural signals, and a terrain awareness display, with an alerting time necessary to prevent controlled flight into terrain events. An acceptable system provides a forward-looking capability and terrain clearance floor protection in areas of operations and surrounding airports of intended use. Such systems generally have: - A navigation system that provides accurate aircraft position (e.g. GPS or equivalent); - A means of displaying aircraft and terrain information; - A means of providing visual and aural signals; - A terrain database(s) for all areas of potential operations and surrounding airports of intended use; If an obstacle database is commercially available and obstacle detection/display functionality is installed, an obstacle database for all areas of potential operations.

FLT 4.3.6

If applicable, the Operator should ensure aircraft in its fleet with a maximum certificated takeoff mass in excess of 5700 kg (12,566 lb), or authorized to carry more than nine passengers, are equipped with a forward-looking windshear warning system.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed aircraft type specifications/system descriptions related to forward-looking windshear warning systems. ☐ Identified/Assessed method(s) used to ensure aircraft are equipped with a forward-looking windshear warning system. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Refer to the IRM for the definition of Forward-Looking Windshear Warning System.

FLT 4.3.7

The Operator shall ensure all aircraft in its fleet are equipped with at least one Emergency Locator Transmitter (ELT) of any type, except as required in FLT 4.3.8. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Examined aircraft type specifications/system descriptions related to ELT. ☐ Identified/Examined method(s) used to ensure aircraft are equipped with any type of ELT. ☐ Other Action (Specify) Click or tap here to enter text.

Guidance Refer to the IRM for the definition of Emergency Locator Transmitter (ELT) which includes the definitions for the types of ELTs. The intent of this provision is to ensure all aircraft, regardless of configuration (passenger, cargo, combi), are equipped with an ELT.

FLT 4.3.8

The Operator shall ensure aircraft in its fleet, for which the individual certificate of airworthiness is first issued after 1 July 2008, are equipped with at least one automatic Emergency Locator Transmitter (ELT).

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Examined aircraft type specifications/system descriptions related to automatic ELT. ☐ Identified/Examined method(s) used to ensure aircraft are equipped with automatic ELT. ☐ Other Action (Specify) Click or tap here to enter text.

FLT 4.3.9

The Operator should ensure all aircraft in its fleet are equipped with at least one automatic Emergency Locator Transmitter (ELT).

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Examined aircraft type specifications/system descriptions related to automatic ELT. ☐ Identified/Examined method(s) used to ensure aircraft are equipped with automatic ELT. ☐ Other Action (Specify) Click or tap here to enter text.

 FLT 4.3.10

If the Operator conducts single-pilot operations, the Operator shall ensure that any aircraft operated by a single pilot under IFR or at night is equipped with: - a serviceable autopilot with at least altitude-hold and heading-select modes; - a headset with a boom microphone or an equivalent communication device; and - means of displaying charts that are readable in all ambient light conditions.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Examined The requirement for equipment on aircraft operated by a single pilot ☐ Identified/Examined method(s) used to ensure aircraft are equipped with autopilot, headset and displaying charts. ☐ Other Action (Specify) Click or tap here to enter text.

| Table 2.1–Onboard Library Specifications |  |  |
| --- | --- | --- |
| The following documents shall be included in the Onboard Library: |  |  |
| General Operating Information |  |  |
| (i) | General operating information to include: - A copy of the air operator certificate (AOC); - A copy of the operations specifications relevant to the aircraft; - If applicable, a copy of the Article 83 bis agreement summary (including an English version); - The Operations Manual (OM). Note: Refer to the IRM for the definitions of Article 83 bis, Article 83 bis Agreement and Article 83 bis Agreement Summary |  |
| Aircraft Operating Information |  |  |
| (ii) | Applicable Aircraft Operating Manual (AOM) and, as a minimum: |  |
|  | (a) | Normal and Emergency Checklists for each operating flight crew member as required by the AFM; |
|  | (b) | Performance tables or access to performance calculations via telecom systems (e.g. ACARS) is acceptable, if completed with appropriate backup procedures; |
|  | (c) | Takeoff performance deviations (e.g. due to inoperative equipment or abnormal situations). |
| (iii) | Minimum Equipment List (MEL) and Configuration Deviation List (CDL); |  |
| (iv) | Aircraft specific weight/mass and balance instructions/data (including loadsheet); |  |
| Areas, Routes and Airport Information |  |  |
| (v) | Flight Plans (OFP and ATS) for each flight; |  |
| (vi) | The applicable departure, navigation and approach charts for use by each operating flight crew member as required by the AFM; |  |
| (vii) | Route and airport instructions and information (flight crew member route guide) for each flight to include, as a minimum: |  |
|  | (a) | Departure airport; |
|  | (b) | Destination airport; |
|  | (c) | En route alternate airports; |
|  | (d) | Emergency airports. |
| (viii) | If applicable, the escape routes used in case of decompression or engine failure in an area of high terrain. |  |
| Other Information |  |  |
| (ix) | Cabin safety and emergency procedures relevant to the flight crew. |  |
| (x) | Dangerous Goods manual or parts relevant to the flight crew, to include information and instructions on the carriage of dangerous goods and action to be taken in the event of an emergency. |  |
| (xi) | Security Manual or parts relevant to the flight crew, including bomb search procedures. |  |
| (xii) | Ground Handling Manual or parts relevant to the flight crew, if required for flight crew to accomplish assigned duties (recommendation only and only applicable to cargo aircraft operations) |  |

| Table 2.2–Operations Manual (OM) Content Specifications |  |  |  |
| --- | --- | --- | --- |
| This table contains the fundamental OM content specifications required to achieve conformance with FLT 1.7.1 and FLT 2.1.10. The table also specifies Section 3 (DSP) provisions that must be addressed in the sections of the OM relevant to flight crew. Note: Specific flight crew policies, guidance, data and/or procedures that must also be addressed in the sections of the OM relevant to flight crew can be found in individual Section 2 provisions and are not duplicated in the table. |  |  |  |
| General Information |  |  | DSP ISARP |
| (i) | General Operations Manual (GOM), to include: |  | None |
|  | (a) | Non-aircraft type related and/or standard operating procedures for each phase of flight, policies, procedures, checklists, descriptions, guidelines, emergency procedures and other relevant information; | None |
|  | (b) | Authorities, duties and responsibilities associated with the operational control of flights; | DSP 1.3.4, 1.3.5 |
|  | (c) | If applicable, guidance that identifies and defines the common flight documents used by the flight crew, the FOO, FOA and/or other personnel responsible for operational control. | None |
| Aircraft Operating Information |  |  | DSP ISARP |
| (ii) | Aircraft Operating Manual (AOM), to include: |  | None |
|  | (a) | Normal, abnormal/non-normal and emergency procedures, instructions and checklists; | None |
|  | (b) | Aircraft systems descriptions, limitations and performance data. | None |
| (iii) | Minimum Equipment List (MEL) and Configuration Deviation List (CDL); |  | None |
| (iv) | Aircraft specific weight/mass and balance instructions/data (including loadsheet); |  | None |
| (v) | Instructions for the computation of the quantities of fuel and oil (if required) to be carried. |  | DSP 4.3.1 |
| Areas, Routes and Airport Information |  |  |  |
| (vi) | Route and airport instructions and information (departure, destination, en route and destination alternates, to include: |  |  |
|  | (a) | Airway manuals and charts, including information regarding communication facilities and navigation aids; |  |
|  | (b) | Airport charts, including the method for determining airport operating minima; |  |
|  | (c) | FMS database; |  |
|  | (d) | Airport and runway analysis manual or documents; |  |
|  | (e) | If applicable, supplemental oxygen requirements; |  |
|  | (f) | If applicable, escape routes used in the event of a decompression or engine failure in an area of high terrain; |  |
|  | (g) | If applicable, procedures for loss of communication between the aircraft and the FOO; |  |
|  | (h) | Instructions for the selection, designation (on the OFP) and protection of departure, en route and destination alternate airports. |  |
|  | (i) | Instructions to address departure if current meteorological reports and forecasts indicate that the destination airport or destination alternate will not be at or above operating minima; |  |
|  | (j) | Instructions to address the continuation of a flight towards an airport of intended landing if the latest available information indicates a landing cannot be accomplished at that airport or at least one destination alternate; |  |
|  | (k) | If applicable, flight monitoring requirements and instructions to ensure the, PIC notifies the operator of en route flight movement or deviations from the OFP; |  |
|  | (l) | If applicable, flight planning considerations that address the continuation of a flight after the failure of the critical engine on a two-engine aircraft and/or the second engine on a three or four engine aircraft; |  |
|  | (m) | The essential information concerning the search and rescue services in the area over which the aircraft will be flown. |  |
|  | (n) | Information regarding RFFS capability available at airports of intended use. |  |
| Training Information |  |  |  |
| (vii) | Training Manual, to include: |  |  |
|  | (a) | Details of all relevant training programs, policies, directives and requirements, including curricula and syllabi, as applicable, for basic operator familiarization, initial qualification, continuing qualification (including recency of experience), re-qualification, aircraft transition or conversion, upgrade to PIC and other specialized training requirements, as applicable; |  |
|  | (b) | Curricula to include: ground training, simulator training, aircraft training, evaluation and certification, line flying under supervision, and any specialized training; |  |
|  | (c) | Comprehensive syllabi to include lesson plans, procedures for training and the conduct of evaluations; |  |
|  | (d) | The training program for the development of knowledge and skills related to human performance (Crew Resource Management/Dispatch Resource Management, CRM/DRM). |  |
| Other Information |  |  |  |
| (viii) | Cabin safety and emergency procedures relevant to the flight crew. |  |  |
| (ix) | Dangerous Goods manual or parts relevant to the flight crew, to include information and instructions on the carriage of dangerous goods and action to be taken in the event of an emergency. |  |  |
| (x) | Security Manual or parts relevant to the flight crew, including bomb search procedures. |  |  |
| (xi) | Ground Handling Manual or parts relevant to the flight crew, if required for flight crew to accomplish assigned duties (recommendation only and only applicable to cargo aircraft operations). |  |  |

| Table 2.3–Flight Crew Qualification Requirements |  |
| --- | --- |
| Fulfillment of the following flight crew certifications, qualifications, training and currency requirements shall be recorded and retained in accordance with ORG 2.6.1, and monitored and considered when assigning flight crew members to duty in accordance with FLT 3.4.1. |  |
| (i) | Licenses/certification, including eligibility to exercise privileges of pilot license/certificate in international operations; |
| (ii) | Specific pilot license/certification limitations (First Officer, relief pilot); |
| (iii) | Specific qualifications (LVP, RVSM, ETOPS/EDTO); |
| (iv) | Equipment qualifications (TCAS/ACAS, GPWS/EGPWS, HGS, HUD/EVS, PBN, PBCS); |
| (v) | Recency-of-experience; |
| (vi) | Medical status, including Medical Certificate; |
| (vii) | Initial training and checking/line check/proficiency check/recurrent training and checking results; |
| (viii) | Right seat qualification; |
| (ix) | Type(s) qualification; |
| (x) | Airport and route competence (including special airports); |
| (xi) | Instructor/evaluator/line check airman qualification; |
| (xii) | CRM/Human Factors training; |
| (xiii) | Dangerous goods training; |
| (xiv) | Security training; |
| (xv) | Accrued flight time, duty time, duty periods and completed rest periods for the purposes of fatigue risk management and compliance with operator or State flight and/or duty time limitations. |

Table 2.4–(Intentionally Open)

| Table 2.5–Route and Airport Knowledge Requirements |  |
| --- | --- |
| Each pilot crew member, in order to conform to the specifications of FLT 2.3.1, shall have adequate knowledge of the following elements related to areas, routes or route segments, and airports to be used in operations: |  |
| (i) | Terrain and minimum safe altitudes; |
| (ii) | Seasonal meteorological conditions; |
| (iii) | Meteorological, communication and air traffic facilities, services and procedures; |
| (iv) | Search and rescue services for the areas over which the aircraft will be flown; |
| (v) | Navigational facilities and procedures, including any long-range navigation procedures associated with the route along which the flight is to take place; |
| (vi) | Procedures applicable to flight paths over heavily populated areas and areas of high air traffic density; |
| (vii) | Airport obstructions, physical layout, lighting, approach aids and arrival, departure, holding and instrument approach procedures and applicable operating minima. |
| Note: That portion of an evaluation relating to arrival, departure, holding and instrument approach procedures may be accomplished in an appropriate training device that is adequate for this purpose. |  |