NOTICE

DISCLAIMER. The information contained in this publication is subject to constant review in the light of changing government requirements and regulations. No subscriber or other reader should act on the basis of any such information without referring to applicable laws and regulations and/or without taking appropriate professional advice. Although every effort has been made to ensure accuracy, the International Air Transport Association shall not be held responsible for any loss or damage caused by errors, omissions, misprints or misinterpretation of the contents hereof. Furthermore, the International Air Transport Association expressly disclaims any and all liability to any person or entity, whether a purchaser of this publication or not, in respect of anything done or omitted, and the consequences of anything done or omitted, by any such person or entity in reliance on the contents of this publication.

© International Air Transport Association. All Rights Reserved. No part of this publication may be reproduced, recast, reformatted or transmitted in any form by any means, electronic or mechanical, including photocopying, recording or any information storage and retrieval system, without the prior written permission from:

Senior Vice President

Operations, Safety and Security

International Air Transport Association

800 Place Victoria

P.O. Box 113

Montreal, Quebec

CANADA H4Z 1M1

IOSA Checklist
ISSM Part I Edition 9

© 2025 International Air Transport Association. All rights reserved.

Montreal—Geneva

# Section 6 —  Ground Handling Operations (GRH)

Applicability Section 6 addresses functions within the scope of ground handling operations and is applicable to an operator that conducts passenger, cargo and/or combi (combined cargo and passenger) aircraft operations. Individual GRH provisions or sub-specifications within GRH provision that: - Begin with a conditional phrase (“If the Operator...”) are applicable if the operator meets the condition(s) stated in the phrase. - Do not begin with a conditional phrase are applicable unless determined otherwise by the Auditor. Functions within the scope of ground handling operations include: - Passenger handling - Baggage handling - Aircraft handling and loading - Aircraft ground movement - Load control - Aircraft fueling - Aircraft de-/anti-icing In this section, non-revenue cargo and mail are addressed in the same way as revenue cargo for the purposes of handling, loading, securing and transporting. COMAT is non-revenue cargo. For the purpose of addressing cargo in this section, mail is considered to be an item of cargo. Therefore, any reference to cargo also includes mail. Where an operator outsources the performance of functions within the scope of ground handling operations to external service providers, the operator retains overall responsibility for such functions, and must demonstrate processes for monitoring the applicable external service providers in accordance with ORG 2.2.1 located in Section 1 of this manual.

General Guidance Definitions of technical terms used in this ISSM Section 6, as well as the meaning of abbreviations and acronyms, are found in the IATA Reference Manual for Audit Programs (IRM). Procedures used in ground operations are defined in the IATA Ground Operations Manual (IGOM), the IATA Airport Handling Manual (AHM), Dangerous Good Regulations (DGR) and in other relevant IATA publications. Due to revision cycle differences, the IATA documents cited above are typically revised at various times during the effective period of an ISSM edition. Accordingly, when an IATA document is revised, it could render an existing reference to specific information in an IATA document to be in error. In such case, the revised IATA document would have to be searched to find the specific information referenced.

## 1 Management and Control

### 1.6 Operational Manuals

GRH 1.6.1

The Operator shall have an Operations Manual, which may be issued in separate parts, that contains the operational policies, processes, procedures and other information necessary for ground handling personnel to perform their duties and be in compliance with applicable regulations, laws, rules and standards of the Operator. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed ground handling OM or, if applicable, separate documents that comprise the OM. ☐ Interviewed responsible management representative(s). ☐ Examined selected sections or parts of the ground handling OM (focus: policies, processes, procedures used by ground handling personnel are included). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Refer to the IRM for the definition of Operations Manual (OM). An OM typically includes guidance that addresses areas generic to all functions within the scope of ground handling operations, as well as parts of the manual that are specific to individual operational functions. Because the scope of ground handling operations is broad and varies by operator, rather than publishing one OM just for ground handling, a smaller operator might choose to incorporate the relevant information into a larger, comprehensive OM. An operator could also choose to issue the information in separate documents that are each specific to the various ground handling operational functions (e.g. passenger handling, baggage handling, aircraft handling). Each individual document would typically contain generic guidance that is applicable to all ground handling operational functions (e.g., organizational policies, general definitions), as well as guidance that is specific to the particular ground handling function or office location (e.g., process descriptions, standard operating procedures, references to the appropriate regulations and IATA manuals). The IGOM contains a core set of operations procedures that may be used by operators in the conduct of ground handling operations. If an operator has external organizations conduct ground handling operations functions, such operator would then be expected to have a monitoring and control process to ensure each external organization either uses the OM of the operator or has its own published OM that fulfills operational safety, security and quality requirements of the operator.

## 2 Training and Qualification

### 2.1 Training Program

GRH 2.1.1

The Operator shall have a process to ensure personnel who perform operational duties in functions within the scope of ground handling operations for the Operator, to include personnel of external service providers, complete: - Initial training prior to being assigned to perform such operational duties; - Recurrent training on a frequency in accordance with requirements of the regulatory authority but not less than once during every 36-month period except recurrent training in dangerous goods as specified in GRH 2.2.1 or GRH 2.2.2; - Re-qualification training applicable to personnel that become unqualified for any reason, prior to being reassigned to perform operational duties. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed training program for ground handling personnel (focus: ensures completion of initial/recurrent training for personnel in all ground handling functions; includes processes that ensure personnel of external service providers complete initial/recurrent training). ☐ Interviewed responsible manager(s) in ground handling operations. ☐ Examined selected initial/recurrent course curricula/syllabi (focus: includes initial and recurrent training programs for all personnel that perform ground handling duties/functions). ☐ Examined initial and recurrent training records of selected personnel (focus: completion of initial and recurrent training). ☐ Examined requalification training records of selected personnel (focus: completion of requalification training). ☐ Other Actions (Specify). Click or tap here to enter text.

Guidance Refer to the Applicability box at the beginning of this section for the functions within the scope of ground handling operations. Requirements for initial and recurrent training apply to all operational ground handling personnel who perform duties within the scope of ground handling operations.

### 2.2 Program Elements

GRH 2.2.1

If the Operator transports dangerous goods as cargo, the Operator shall have a process to ensure ground handling personnel complete dangerous goods training, to include initial training and recurrent training within 24 months of previous training in dangerous goods. Such training shall be completed by personnel that perform operational duties in the following functions within the scope of ground handling operations: - Passenger handling; - Baggage handling; - Aircraft loading; - Load control. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed dangerous goods training program (focus: defines DG training requirements for all cargo handling personnel based on specific assigned responsibilities/duty functions). ☐ Interviewed responsible manager(s) in ground handling operations. ☐ Examined applicable initial/recurrent dangerous goods training curricula and syllabi (focus: subject areas appropriate for personnel based on specific responsibilities/duty functions). ☐ Examined training records of selected personnel (focus: completion of required training as appropriate for assigned responsibilities/duty functions). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The curriculum for dangerous goods training is determined by the operator and may vary depending on specific responsibilities and duty function(s). Recurrent training in dangerous goods is completed within a validity period that expires 24 months from the previous training to ensure knowledge is current, unless a shorter period is defined by a competent authority. However, when such recurrent training is completed within the final 3 months of the 24-month validity period, the new validity period may extend from the month on which the recurrent training was completed until 24 months from the expiry month of the current validity period. If such recurrent training is completed prior to the final three months of the validity period, the new validity period would extend 24 months from the month the recurrent training was completed Refer to DGR 1.5 and Appendix H.6 for guidance that includes adapted task lists for well-defined job functions.

GRH 2.2.2

If the Operator does not transport dangerous goods as cargo, the Operator shall have a process to ensure ground handling personnel receive dangerous goods training, to include initial training and recurrent training within 24 months of previous training in dangerous goods. Such training shall be completed by personnel that perform operational duties in the following functions within the scope of ground handling operations: - Passenger handling; - Baggage handling; - Aircraft loading; - Load control. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed dangerous goods training program: (focus: defines DG training requirements for all cargo handling personnel based on specific assigned responsibilities/duty functions). ☐ Interviewed responsible manager(s) in ground handling operations. ☐ Examined selected initial/recurrent training curricula/syllabi (focus: subject areas appropriate for personnel based on specific responsibilities/duty functions). ☐ Examined training records of selected personnel (focus: completion of required training as appropriate for assigned responsibilities/duty functions). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance When an operator does not transport dangerous goods as cargo (i.e. a “no-carry” operator), dangerous goods training is still required for ground handling personnel to ensure prohibited dangerous goods are recognized and are not loaded onto an aircraft. Dangerous goods training would be structured to provide the requisite knowledge to permit ground handling personnel to recognize prohibited dangerous goods (whether labeled or not labeled), ensure such dangerous goods are not inadvertently loaded on an aircraft and apply emergency action in the event of contamination or a spill. The curriculum for dangerous goods training is determined by the operator and may vary depending on specific responsibilities and duty function(s). Refer to DGR 1.5 and Appendix H.6 for guidance that includes adapted task lists for well-defined job functions.

GRH 2.2.3

The Operator shall have a process to ensure initial and recurrent training or recurrent assessment completed by applicable ground handling personnel in accordance with GRH 2.1.1 addresses the following areas of operations, as applicable to ground handling duties or function(s) performed: - Passenger services; - Ramp services; - Load control; - Aircraft fueling; - Aircraft ground de-/anti-icing. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed training program for ground handling personnel (focus: training program addresses all specified operational areas). ☐ Interviewed responsible manager(s) in ground handling operations. ☐ Examined selected initial/recurrent/requalification training curricula/syllabi (focus: training addresses all specified operational areas). ☐ Examined initial/recurrent/requalification training records of selected personnel (focus: completion of training appropriate for individual duties/functions performed). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance For additional guidance refer to AHM 1110, Ground Operations Training Program.

GRH 2.2.5

The Operator shall have a process to ensure training for ground handling personnel assigned to perform aircraft fueling as specified in GRH 2.2.3 includes the following training elements: - Safe operation of equipment; - Emergency procedures; - Fuel spillage avoidance response; - Aircraft fueling and defueling procedures; - Aircraft-specific training. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed training program for ground handling personnel (focus: program includes the specified training elements associated with aircraft fueling operations). ☐ Interviewed responsible manager(s) in ground handling operations. ☐ Examined initial/recurrent/requalification training curricula/syllabi (focus: curricula/syllabi address the specified training elements for aircraft fuelling). ☐ Examined selected initial/recurrent/requalification training records (focus: personnel have completed training appropriate to operational functions performed). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Refer to the IATA Guidance Material on Standard Into-Plane Fuelling Procedures as applicable to functions directly involved in aircraft fuelling operations.

## 3 Ground Handling Operations

### 3.2 Airside Operations

GRH 3.2.2

The Operator shall ensure aircraft arrival procedures are in place that are completed prior to aircraft arrival at the assigned parking gate or stand. Such procedures shall ensure: - The ramp area surface is inspected and is free of: (a) Debris that could cause foreign object damage (FOD); (b) Contamination that could be hazardous to aircraft movement. - The aircraft movement path is clear of objects and obstacles; - Personnel not involved in the aircraft arrival are positioned outside the equipment restraint area (ERA); - Required GSE is available and positioned clear of the ERA; - The aircraft docking guidance system is operational or, if applicable, marshalling personnel are in place; - If applicable, wing walkers and/or other applicable personnel are present. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed requirement for implementation of safety procedures in the conduct of airside operational activities. ☐ Interviewed responsible manager(s) in ground handling operations. ☐ Observed aircraft arrival operations (focus: procedures for aircraft arrival are implemented as published in the OM). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Refer to the IRM for the definition of Equipment Restraint Area (ERA) and Foreign Object Debris/Damage (FOD). As specified in item (i) (b), snow and ice are ramp surface contaminants that can be hazardous to aircraft ground movement. Documented procedures in accordance with IGOM 4.1.1, 4.1.2.1 and 4.1.3.1 will typically demonstrate documental conformity with the specifications in this provision.

### 3.3 Load Control

GRH 3.3.1

The Operator shall ensure a Load Control system is in place that provides for: - Aircraft weight and balance conditions that are correct and within limits; - Aircraft loaded in accordance with applicable regulations and specific loading instructions for the flight; - Dissemination of dangerous goods and other special load information applicable to each flight; - Information, to include last minute changes, that is in agreement with the actual load on the aircraft and presented on a final load sheet. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed Load Control system. ☐ Interviewed responsible manager(s) in load control operations. ☐ Examined checklists/procedures used in the load control process. ☐ Observed load control operations (focus: load control system includes functions necessary to address aircraft load, weight/balance calculation, production of final load sheet). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Refer to the IRM for the definitions of Load, Load Control, Special Load and Weight and Balance Manual (W&BM). A load planning system typically entails, as a minimum: - Assemblage of all data relating to the aircraft load (originating and en route stations); - Planning of the load for ready accessibility; - Planning of special loads according to restrictions, maximum quantities, separation and segregation requirements; - Consideration of center of gravity parameters affecting aircraft fuel consumption. Guidance may be found in AHM 551 and 590.

GRH 3.3.4

If the Operator transports dangerous goods as cargo, the Operator shall ensure a process is in place to provide the pilot-in-command (PIC), as soon as practicable prior to departure of the aircraft, with accurate and legible written information pertaining to dangerous goods on board the aircraft to be transported as cargo. Such notification shall include the following: - If applicable, Air Waybill number; - Proper shipping name and/or UN/ID number; - Class or division, and subsidiary hazard(s) corresponding to the label(s) applied, and for Class 1, the compatibility group; - If applicable, packing group; - For non-radioactive material, number of packages, exact loading location and, as required, net quantity or, if applicable, gross weight of each package, except: △ a) For UN 1845: carbon dioxide, solid (dry ice), UN number, proper shipping name, classification, total quantity in each aircraft hold and offload airport are required; b) For UN 3480 (Lithium-ion batteries) and UN 3090 (lithium-metal batteries): At the option of the Operator, only the UN number, proper shipping name, class, total quantity at each loading location, and whether the package must be carried on a cargo only aircraft are required. UN 3480 (Lithium-ion batteries) and UN 3090 (lithium-metal batteries) carried under a State exemption must meet all of the requirements of iv) and v). - For radioactive material, number and category of packages, overpacks or freight containers, exact loading location and, as applicable, transport index and dimensions for each package; - Any restriction for transport on cargo aircraft only; - Airport of unloading; - If applicable, dangerous goods transported under a state exemption; - If applicable, ULD Identification Code. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed load control process to provide PIC with information pertaining to onboard dangerous goods as cargo. ☐ Interviewed responsible manager(s) in load control operations. ☐ Examined documents (e.g. NOTOC) that confirm dangerous goods information was provided to PIC (focus: use of checklist/form that conforms to the specifications stated in the provision). ☐ Observed load control operations (focus: load control system includes process/method for providing applicable dangerous goods information to PIC). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Refer to the IRM for the definitions of NOTOC (Notification to Captain) and State. Information pertaining to dangerous goods on board the aircraft is typically presented to the PIC in a notification called the NOTOC (notification to the captain). The NOTOC contains the detailed information (as specified in this provision) relative to all dangerous goods loaded on the aircraft as cargo. Information contained in the NOTOC may also be used: - For emergency response to an accident or incident involving dangerous goods on board; - To provide to air traffic services in the event of an in-flight emergency. In the event the NOTOC is of such a size as to make in-flight radiotelephony transmission impracticable in an emergency situation, a summary of the information is typically provided to the PIC (NOTOC Summary), which contains at least the quantities and classes or division of dangerous goods in each cargo compartment. Guidance may be found in DGR 9.5

### 3.4 Aircraft Loading

GRH 3.4.1

The Operator shall have aircraft loading procedures in the OM that ensure: - The cargo hold is inspected before loading to: (a) Check for damage (b) Ensure it is empty of other than documented transit load items. - The aircraft is loaded: (a) In accordance with written loading instructions; (b) In a manner that satisfies weight and balance requirements. - The load is secure and will not move during the flight; - If applicable, ULD locks are extended and locked. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed aircraft loading procedures. ☐ Interviewed responsible manager(s) in ground handling operations. ☐ Examined examples of documented aircraft loading instructions. ☐ Observed aircraft loading operations (focus: aircraft loaded in accordance with loading instructions/weight/balance requirements). ☐ Interviewed personnel that perform aircraft loading. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Refer to IGOM 4.5.9, 5.6.1 and 5.6.2, as well as AHM 514 and 590 for additional guidance.

GRH 3.4.3

If the Operator transports dangerous goods as cargo, the Operator shall ensure procedures are in place for the transportation of dangerous goods to/from an aircraft and the loading and securing of dangerous goods on an aircraft in a manner that: - Prevents damage to packages and containers during aircraft loading and unloading; - Provides for separation and segregation in accordance with applicable requirements; - Prevents any movement in the aircraft. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed procedures for the transportation of dangerous goods to/from an aircraft and and the loading/securing of dangerous goods on an aircraft. ☐ Interviewed responsible manager(s) in ground handling operations. ☐ Observed transportation of cargo to/from aircraft and the loading and securing of dangerous goods (focus: handling of dangerous goods to prevent damage, prevent movement in the aircraft, and maintain separation). ☐ Interviewed personnel that perform aircraft loading and securing of cargo shipments. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Refer to DGR 9.3 for guidance that addresses the the transportation, loading and securing of dangerous goods, and to DGR 10.9 for guidance that addresses securing and separation of radioactive material. Refer to IGOM 4.5.7.7 for guidance that addresses securing of dangerous goods

GRH 3.4.4

If the Operator transports dangerous goods as cargo, the Operator shall ensure procedures are in place that assure, when a dangerous goods package or shipment appears to be damaged or leaking: - The package or shipment is prevented from being loaded into an aircraft; - If already loaded, the package or shipment is removed from an aircraft; - In the case of leakage, an evaluation is conducted to identify and prevent from transport any baggage, cargo, transport devices or other items that may have become contaminated. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed procedures for handling/addressing leaking/damaged dangerous goods shipments ☐ Interviewed responsible manager(s) in load control operations. ☐ Examined records/documents that illustrate handling of leaking/damaged dangerous goods shipments. ☐ Observed aircraft loading operations (focus: procedures for addressing dangerous goods packages/shipments that appear to be leaking or damaged). ☐ Interviewed personnel that perform aircraft loading. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Refer to DGR 9.3, 9.4 and 10.9, which contain guidance that addresses apparent damage to dangerous goods shipments.

GRH 3.4.5

If the Operator transports dangerous goods as cargo, the Operator shall ensure procedures are in place that require, when an aircraft has been contaminated by dangerous goods leakage: - Hazardous contamination is removed from the aircraft without delay; - In the case of radioactive contamination, arrangements are made to take the aircraft out of service for evaluation by appropriately qualified personnel.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed procedures for addressing aircraft contaminated by leakage of dangerous goods. ☐ Interviewed responsible manager(s) in ground handling operations. ☐ Examined guidance/checklists used for dealing with an aircraft contaminated by leakage of dangerous goods. ☐ Other Actions (Specify) Click or tap here to enter text.

GRH 3.4.10

If the Operator transports dangerous goods as cargo, the Operator shall ensure procedures are in place that prevent dangerous goods from being carried in an aircraft cabin occupied by passengers, except as permitted by the Authority or the DGR. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed procedure(s) for preventing the transport of dangerous goods in an aircraft cabin occupied by passengers, except as permitted. ☐ Interviewed responsible manager(s) in load control operations. ☐ Examined guidance/checklists used to ensure dangerous goods are not transported in an aircraft passenger cabin, except as permitted. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance In general, dangerous goods are prohibited from being transported in an aircraft cabin occupied by passengers. Limitations and exceptions are specified in DGR Sections 2 and 9.

GRH 3.4.11

If the Operator transports dangerous goods as cargo, the Operator shall ensure procedures are in place that prevent dangerous goods from being carried on the aircraft flight deck, except as permitted by the Authority or the DGR. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed procedure(s) for preventing the carriage of dangerous goods on the aircraft flight deck, except as permitted. ☐ Interviewed responsible manager(s) in load control operations. ☐ Examined guidance/checklists used to ensure dangerous goods are not carried on the aircraft flight deck, except as permitted. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance In general, dangerous goods are prohibited from being transported on the flight deck of an aircraft. Limitations and exceptions are specified in DGR Sections 2 and 9.

## 4 Special Aircraft Ground Handling Operations

### 4.1 Aircraft Fueling

GRH 4.1.1

The Operator shall have a process to ensure fuel suppliers are maintaining standards of fuel safety and quality acceptable to the Operator and fuel delivered and loaded onto aircraft is: - Of the correct grade and specification for each aircraft type; - Free from contamination. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed process for monitoring of fuel quality at all locations where aircraft are refueled. ☐ Interviewed responsible manager(s) in ground handling operations. ☐ Examined selected quality control inspection reports (focus: fuel supply quality management). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The intend of this standard is to ensures fuel is stored, handled and serviced in accordance with accepted standards. Approved fuel specifications are contained in each aircraft manual. To ensure fuel corresponds to the specifications and grade of product necessary for the applicable aircraft type(s), a control process is typically in place at each location where the operator has aircraft fueling operations. Such process ensures the existence of periodic inspections of critical aspects of the fuel supply system at each applicable location, to include, as a minimum: - Fuel facilities; - Safety and quality procedures; - Performance levels of personnel. Processes for ensuring fuel is of the correct grade and free of contamination may be documented in maintenance, ground operations or flight operations manuals, or in a combination thereof. If the Operator uses biofuels, additional procedures would typically address the related specific requirements (i.e. dedicated infrastructures and blending requirements). Additional guidance may be found in the IFQP (IATA Fuel Quality Pool) Quality and Safety Procedures, and in the AEA Recommendations for De-icing/Anti-icing of Aircraft on the Ground.

GRH 4.1.2

The Operator shall ensure, during fuelling operations with passengers embarking, on board or disembarking the aircraft, procedures are in place that provide for the designation of a person with responsibility for fueling operations and specify the method(s) by which that responsible person: - Communicates with the flight crew or other qualified personnel on board the aircraft; - Provides notification to the flight crew or other qualified personnel on board the aircraft when a hazardous condition or situation has been determined to exist; - Provides notification to the flight crew or other qualified personnel on board the aircraft and other appropriate personnel engaged in aircraft ground handling activities when fueling is about to begin and has been completed. (GM) Note: Notification when fueling is about to begin and has been completed as in iii. maybe replaced by an equivalent procedural means which ensure the flight crew or other qualified personnel on board the aircraft are aware of fueling operations and are in a position to, if necessary, perform an expeditious evacuation of the aircraft.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed procedures for communication between ground and onboard personnel during aircraft fuelling operations. ☐ Interviewed person(s)/manager(s) responsible for fuelling operations. ☐ Interviewed selected aircraft fuelling supervisory personnel. ☐ Observed aircraft ground handling operations (focus: establishment of method for groundaircraft communication during aircraft fuelling operations). ☐ Coordinated with flight operations and cabin operations to verify complementary procedures for communication with ground personnel during aircraft fuelling operations. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Ground handling personnel would typically need to have a clear understanding of all required communication procedures and have the ability to execute such procedures in an expeditious manner should a dangerous situation develop. The specification in item ii) may be satisfied by either: - Equivalent procedural means, acceptable to the State and applicable authorities, that would permit the flight crew or other qualified persons to be aware of the start and completion of fuelling operations, or - Procedures established by the operator that would ensure qualified personnel on board the aircraft are continuously in a position to perform an expeditious evacuation of the aircraft for any reason, including a fuel spill or fire. Suitable methods of communication with the flight crew or other qualified personnel on board the aircraft include use of the aircraft inter-communication system, direct person-to-person contact or other methods that ensure direct and timely communication. Use of the aircraft inter-communication system to maintain continuous two-way communication during fuelling operations is not a requirement. The following roles and main responsibilities of participating personnel typically involve the following: - Other qualified personnel on board the aircraft remain at a specified location during fuelling operations and are capable of handling emergency procedures associated with fire protection and fire-fighting, and communicating, initiating and directing an evacuation as specified by the operator. Such personnel are qualified by the operator to assume and perform flight crew and/or cabin crew responsibilities and duties during fuelling operations with passengers on board the aircraft. - Flight crew or other qualified personnel on board the aircraft (e.g. cabin crew) are in a position to perform an expeditious evacuation of the aircraft and are aware of the state of fuelling operations. Such qualified personnel are responsible for two-way communication being established and maintained with personnel that are responsible for fuelling operations. - A person is responsible for supervising fueling operations and maintaining two-way communication with the flight crew or other qualified personnel on board the aircraft (via the aircraft interphone communication system or other suitable means). Such person ensures an appropriate notification to qualified personnel on board the aircraft when fueling is about to begin, has been completed and when a hazardous condition or situation has been determined to exist. Such person also is responsible for communication with other appropriate personnel engaged in aircraft ground handling activities to ensure safe ramp conditions are maintained during fueling operations with passengers on board. - Other appropriate personnel engaged in aircraft ground handling activities are responsible for maintaining safe ramp conditions during fueling operations, which includes, but is not limited to: - Embarkation or disembarkation path is not obstructed by GSE; - Aircraft handling operations do not create a hazard or obstruct emergency exits; - Ground areas beneath nominated exit doors are kept clear of any obstructions that would impede an emergency evaluation. Criteria that identify the commencement of aircraft fueling typically include conditions such as the fueling vehicle being in position and bonded to the aircraft and the fueling hose-end nozzle being connected to the aircraft fuel adapter. Criteria that identify completion of aircraft fueling typically include conditions such as the hose-end nozzle disconnected from the aircraft fuel adapter, the hydrant pit/inlet hose is disconnected (if applicable), bonding cables are detached, the fuel receipt (slip) is processed, there are no leaks or spills identified and the fueling vehicle is ready to leave its position (as applicable based on fueling vehicle operator's walk-around check performed). Additional guidance may be found in AHM 462 Item 9.5, as well as the ICAO Airport Services Manual, Document 9137 (ASM), Part 1.

GRH 4.1.5

The Operator shall ensure safety procedures associated with aircraft fueling operations are in place that assure, during fueling operations with passengers on board the aircraft: - The ground area beneath aircraft exit doors that have been designated for rapid deplaning or emergency evacuation is kept clear of obstructions; - Where a boarding bridge is in use, an interior access path is maintained from the aircraft to the terminal; - Where a passenger boarding bridge is not in use, aircraft passenger steps or an alternate means of emergency evacuation is in place. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed safety procedures for aircraft fueling operations (focus: specifications of this standard are included/addressed in fueling procedures) ☐ Interviewed responsible manager(s) in ground handling operations. ☐ Interviewed selected aircraft fuelling supervisory personnel. ☐ Observed aircraft ground handling operations (focus: implementation of safety procedures during aircraft fueling operations). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Refer to the IRM for the definition of Integral Airstairs. When deployed, integral airstairs are acceptable as an alternate means of emergency evacuation. Refer to IGOM 3.2.3 for GSE positioning for aircraft refueling operations. Additional guidance may be found in AHM 462, as well as the ICAO ASM, Part 1.

### 4.2 Aircraft De-/Anti-icing

GRH 4.2.1

If the Operator conducts flights from any airport when conditions are conducive to ground aircraft icing, the Operator shall have a De-/Anti-icing Program, which, if applicable, is approved by the Authority and, as a minimum: - Ensures adherence to the Clean Aircraft Concept; - Defines responsibilities within the Program; - Addresses applicable locations within the route network; - Defines areas of responsibility; - Specifies technical and operational requirements; - Specifies training and qualification requirements; - Is applicable to external service providers that perform de-/anti-icing functions for the Operator. (GM) Note: The specifications of this provision are also applicable for flights such as ferry, maintenance check/test and training.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed approved aircraft de-/anti-icing program. (focus: all applicable locations within the route network are addressed; flight such as: ferry, maintenance check/test and training are accounted for). ☐ Interviewed responsible manager(s) in ground handling operations. ☐ Examined de-/anti-icing program at selected airports. (focus: de-/anti-icing program requirements and areas of responsibilities are addressed as per selected airport’s local conditions). ☐ Examined reports that detail past de-/anti-icing operations at selected airports. (focus: de-/anti-icing operations performed by external service providers are continuously reported to the operator). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Refer to the IRM for the definitions of De-/Anti-icing Program and Clean Aircraft Concept. A de-/anti-icing program would address not only commercial and non-commercial operations at an applicable airport but, operations such as positioning flights, test flights, training flights and etc. The scope and details of a de-/anti-icing program would typically be commensurate with the frequency and complexity of operations at airports with the potential for ground icing conditions. Additional guidance may be found in ICAO Doc 9640-AN/940, Manual of Aircraft Ground De-icing/Anti-icing Operations, Chapter 3, and in SAE AS6285, Aircraft Ground Deicing/Anti-Icing Program. The latter two are used as the basis for inspections conducted under the IATA De-Icing/Anti-Icing Quality Control Pool (DAQCP).